SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONICS

ENGINEERING (BSEEE)

DIGITAL CONTROL 1

Lab 1:

MICROCONTROLLER-BASED DIGITAL CONTROL OF ROBOT CAR DRIVEN WITH BLDC MOTORS

BY

Edward Mwongela – 142092

Group members

Harry Boto

Arnold Injeni

Samuel Mwichigi

Trevor Ayimba

Lecturer: Prof. Jackson Mbuthia

30th March 2025

Contents

List of figures	2

Objective:	3

Learning Outcome:	3

Introduction	4

Components Required	4

1.	Robot car assembly	4

2.	Experiment Design - Simulate system response to various control strategies (e.g., Lag, Lead, PID, state feedback)	7

Methodology	33

Procedure	33

Discussion	35

6.2.1 How do the theoretical simulations compare with practical results?	35

6.2.2 How does closing the loop with unity feedback change the response of the robot car?  Possible Improvements:	35

6.2.3 How does closing the loop with unity feedback and adding the forward PID controller change the response of the robot car?	35

6.2.4 How does closing the loop with unity feedback, adding the forward PID controller and an object in section F - G change the response of the robot car?	36

Challenges and Recommendations	37

Appendix	38

References	41

## List of figures

Figure 1. Brushless DC Motor – 3 Phase Star Connected Armature (Stator or Rotor)	4

Figure 2. Electronic Speed Controller	5

Figure 3. Motor-ESC Wiring Diagram	6

Figure 4. Simple Flat Robot Car Racing Track – Marked out in Power and Control Lab	33

# Objective:

- Implement closed-loop digital speed and direction control for a two-wheeled BLDC motor-driven robot car.

- Use python code to simulate system response to various control strategies (e.g., Lag, Lead, PID, state feedback)

- Use python code to simulate driving the robot car with on board camera orientation and obstacle detection and avoidance control.

- Implement real time microcontroller-based control laws for the motor-driven robot car (e.g., Lag, Lead, PID)

- Compare the results obtained in 1 and 2 above.

- Implement real time microcontroller-based control laws for the robot car driven round a flat track and include obstacle detection and avoidance.

# Learning Outcome:

- Apply experimental procedures to assess control laws that optimize the performance of the robot car.

- Apply experimental procedures to determine suitable parameters for the robot car transfer function.

# Introduction

## Components Required

- Microcontroller (e.g., Raspberry PI)

- Two BLDC motors with drivers (e.g., ESCs)

- Motor encoders (for speed feedback control)

- Power supply (LiPo or Lithium-Ion battery)

- Sensors (e.g., on-board camera for orientation and obstacle avoidance)

- Computer for code development.

## Robot car assembly

The Robot car uses two brushless motors shown in Fig. 1 to drive two front wheels and 2

Electronic Speed Controllers (ESC) are shown in Fig. 2 to drive the front wheels. The back

wheels are assembled to allow free movement.

Figure 1. Brushless DC Motor – 3 Phase Star Connected Armature (Stator or Rotor)

| No. of Cells: 2 - 3 Li-Poly | 2 - 3 Li-Poly 6 - 10 NiCd/NiMH |
| --- | --- |
| Kv: | 1000 RPM/V |
| Max Efficiency: | 80% |
| Max Efficiency Current: | 4 - 10A (>75%) |
| No Load Current: | 0.5A 10V |
| Resistance: | 0.090 ohms |
| Max Current: | 13A for 60S |
| Max Watts: | 150W |
| Weight: | 52.7 g / 1.86 oz |
| Size: | 28 mm dia x 28 mm bell length |
| Shaft Diameter: | 3.2 mm |
| Poles: | 14 |
| Model Weight: | 300 - 800g / 10.5 - 28.2 oz |

Table 1. A2212/13T technical data

A small yet powerful motor for planes up to eight hundred grams (28 oz) using 3 li-poly cells. The motor features a 3.2mm hardened steel shaft, dual ball bearings, and has 3.5mm gold spring male connectors already attached and includes 3 female connectors for your speed control. It includes a collet type prop adapter and radial motor mount. Mounting holes have 16mm and 19mm spacing on centers and are tapped for 3mm (M3) screws. An excellent higher-powered replacement for geared Speed 400-480 motors in slow-flying or 3D planes that require a larger 10" propeller. The recommended prop is 10 x 5 on 3 li-poly cells. Like WelgardA2212-13, AXI Gold A2212/26, WelgardC2830-12, E-Flite Park 400. Great replacement motor for a 1/2A Texaco engine.[1]

Figure 2. Electronic Speed Controller

The motor shown in Fig. 1 has a maximum speed of 1000Kv * DC Supply Voltage. To obtain optimum torque over a wide speed range when using the ESC, it is necessary to have one ratio gear to reduce the maximum wheel speed to a third of the maximum BLDC motor speed. This will produce an equivalent maximum Robot car wheel speed of approximately 11 m/s.

The motor has three input wires to supply three windings connected in star configuration as shown on Fig. 1. The ESC is a motor driver that produces a variable voltage to supply the motor. A PWM pulse is used to control the motor windings voltage Van rms value and frequency to produce variable wheel thrust/speed and to implement control concepts generated from code which is run on a microcontroller.

The Motor-ESC wiring diagram is shown in Fig. 3

Figure 3. Motor-ESC Wiring Diagram

5V- Driver board internal output voltage

M - Motor speed pulse signal output, 5V pulse signal.

(i) Control port

Z/F- Rotating direction control ports. Connect “5V” high level or no connect is Forward direction, connect 0 V low level or connect to GND is reverse direction VR ----Speed/thrust control port. Analog voltage linear speed regulation 0.1v-5V. The input resistance is 20K Ohm, connect with GND for input PWM speed regulation [2]

PWM frequency:1- 20KHZ.

Duty cycle 0-100% GND—Used for Drive board internal control.

(ii) Power port

MA - motor phase A

MB - motor phase B

MC - motor phase C

GND - DC-

VCC - DC +

## Experiment Design - Simulate system response to various control strategies (e.g., Lag, Lead, PID, state feedback)

BLDC Motor Model Kv:1000

Max Efficiency: 80%

Max Efficiency Current: 4-10A (>75%)

Current Capacity: 12A/60s

No Load Current 10V: 0.5A

Speed Kv*Voltage 0r 10,000rpm 10V

4.2.1 Given the system parameters provided above, write python code to simulate the open-loop response of your robot car and provide a plot of your motor current ia(t) Amps, rotor speed ω(t) rad/s and road speed v(t) m/s            10 Marks

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000                # rpm/V

kt = 60 / (2 * np.pi * Kv)  # Nm/A

V_supply = 10            # Volts

R_motor = 0.5            # Ohms (assumed)

no_load_current = 0.5    # Amps

wheel_radius = 0.032     # meters

# Time vector

t = np.linspace(0, 2, 500) # 0 to 2 seconds

ia = np.full_like(t, no_load_current)  # constant no-load current

# Rotor speed in RPM then converted to rad/s

rpm = Kv * V_supply

omega = (rpm * 2 * np.pi) / 60  # rad/s

# Road speed = wheel angular velocity * radius

v = omega * wheel_radius

# Expand to match array length

omega_array = np.full_like(t, omega)

v_array = np.full_like(t, v)

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, label='Motor Current ia(t)', color='r')

plt.ylabel('Current (A)')

plt.grid(True)

plt.subplot(3, 1, 2)

plt.plot(t, omega_array, label='Rotor Speed ω(t)', color='b')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v_array, label='Road Speed v(t)', color='g')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.tight_layout()

plt.show()

4.2.2 Modify the python code written in 4.2.1 to limit the motor current to a maximum of 10A and provide a plot of your motor current ia(t) Amps, rotor speed ω(t) rad/s and road speed v(t) m/s            5 Marks.

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000

kt = 60 / (2 * np.pi * Kv)

V_supply = 10

R_motor = 0.5

max_current = 10

no_load_current = 0.5

wheel_radius = 0.032

# Time vector

t = np.linspace(0, 2, 500)

# Simulate motor current ramping under load

ia = np.linspace(no_load_current, 15, len(t))

ia = np.clip(ia, 0, max_current)  # Limit current to 10A

# Assume torque = kt * ia

torque = kt * ia

# Angular speed = proportional to torque (simple linear approx here)

omega = (Kv * V_supply * 2 * np.pi) / 60  # rad/s constant

omega_load_effect = omega * (ia / max_current)  # reduce speed when current is limited

# Vehicle speed

v = omega_load_effect * wheel_radius

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, label='Motor Current ia(t)', color='r')

plt.axhline(max_current, color='k', linestyle='--', label='Max Current')

plt.ylabel('Current (A)')

plt.grid(True)

plt.legend()

plt.subplot(3, 1, 2)

plt.plot(t, omega_load_effect, label='Rotor Speed ω(t)', color='b')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v, label='Road Speed v(t)', color='g')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.tight_layout()

plt.show()

4.2.3 Given the system parameters provided above, write python code to simulate the closed-loop response of your robot car and provide a plot of your motor current ia(t) Amps, rotor speed ω(t) rad/s and road speed v(t) m/s            10 Marks

#  4.2.3 CLOSED-LOOP RESPONSE

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000  # rpm/V

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5

wheel_radius = 0.032

V_max = 10  # Max voltage

# Simulation setup

dt = 0.01

t = np.arange(0, 2, dt)

v_ref = 1.5  # Desired speed in m/s

Kp = 5  # Proportional gain

# Initialize arrays

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

# Simulation loop

for i in range(1, len(t)):

# Feedback control (voltage)

error = v_ref - v[i-1]

voltage = Kp * error

voltage = np.clip(voltage, 0, V_max)  # Clamp to motor voltage range

# Rotor speed = Kv * voltage

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60  # rad/s

# Current: I = V / R (basic ohmic model)

ia[i] = voltage / R_motor

# Car speed = ω * r

v[i] = omega[i] * wheel_radius

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, label='Motor Current ia(t)', color='r')

plt.ylabel('Current (A)')

plt.grid(True)

plt.subplot(3, 1, 2)

plt.plot(t, omega, label='Rotor Speed ω(t)', color='b')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v, label='Road Speed v(t)', color='g')

plt.axhline(v_ref, color='k', linestyle='--', label='Reference Speed')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.legend()

plt.tight_layout()

plt.show()

4.2.4 Modify the python code written in 4.2.3 to limit the motor current to a maximum of 10A and provide a plot of your motor current ia(t) Amps, rotor speed ω(t) rad/s and road speed v(t) m/s             5 Marks.

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5

wheel_radius = 0.032

V_max = 10

I_max = 10

V_limit = I_max * R_motor  # Max allowed voltage due to current limit

# Simulation setup

dt = 0.01

t = np.arange(0, 2, dt)

v_ref = 1.5  # m/s

Kp = 5

# Initialize arrays

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

# Simulate

for i in range(1, len(t)):

# Feedback voltage

error = v_ref - v[i-1]

voltage = Kp * error

voltage = np.clip(voltage, 0, min(V_max, V_limit))  # Limit voltage to satisfy current limit

# Motor current

ia[i] = voltage / R_motor

# Rotor speed

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60

# Road speed

v[i] = omega[i] * wheel_radius

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, label='Motor Current ia(t)', color='r')

plt.axhline(I_max, color='k', linestyle='--', label='Max Current')

plt.ylabel('Current (A)')

plt.grid(True)

plt.legend()

plt.subplot(3, 1, 2)

plt.plot(t, omega, label='Rotor Speed ω(t)', color='b')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v, label='Road Speed v(t)', color='g')

plt.axhline(v_ref, color='k', linestyle='--', label='Reference Speed')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.legend()

plt.tight_layout()

plt.show()

4.2.5 Given the system parameters provided above, write python code to simulate the closed-loop response with forward PID of your robot car and provide a plot of your motor current ia(t) Amps, rotor speed /s and road speed v(t) m/s            10 Marks.

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5

wheel_radius = 0.032

V_max = 10

# PID gains

Kp = 6

Ki = 20

Kd = 0.05

# Setup

dt = 0.01

t = np.arange(0, 2, dt)

v_ref = 1.5

# Init arrays

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

# PID terms

integral = 0

prev_error = 0

for i in range(1, len(t)):

# Error

error = v_ref - v[i-1]

integral += error * dt

derivative = (error - prev_error) / dt

prev_error = error

# PID control

voltage = Kp * error + Ki * integral + Kd * derivative

voltage = np.clip(voltage, 0, V_max)

# Motor current

ia[i] = voltage / R_motor

# Rotor speed

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60

# Vehicle speed

v[i] = omega[i] * wheel_radius

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, color='r', label='Motor Current ia(t)')

plt.ylabel('Current (A)')

plt.grid(True)

plt.subplot(3, 1, 2)

plt.plot(t, omega, color='b', label='Rotor Speed ω(t)')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v, color='g', label='Road Speed v(t)')

plt.axhline(v_ref, color='k', linestyle='--', label='Reference Speed')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.legend()

plt.tight_layout()

plt.show()

4.2.6 Modify the python code written in 4.2.5 to limit the motor current to a maximum of 10A and provide a plot of your motor current ia(t) Amps, rotor speed ω(t) rad/s and road speed v(t) m/s             5 Marks.

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5

wheel_radius = 0.032

V_max = 10

I_max = 10

V_limit = I_max * R_motor

# PID gains

Kp = 6

Ki = 20

Kd = 0.05

# Time setup

dt = 0.01

t = np.arange(0, 2, dt)

v_ref = 1.5

# Initialize

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

integral = 0

prev_error = 0

for i in range(1, len(t)):

error = v_ref - v[i-1]

integral += error * dt

derivative = (error - prev_error) / dt

prev_error = error

# PID controller

voltage = Kp * error + Ki * integral + Kd * derivative

# Limit voltage to both motor range and current limit

voltage = np.clip(voltage, 0, min(V_max, V_limit))

# Current

ia[i] = voltage / R_motor

# Rotor speed

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60

# Road speed

v[i] = omega[i] * wheel_radius

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, ia, label='Motor Current ia(t)', color='r')

plt.axhline(I_max, color='k', linestyle='--', label='Max Current')

plt.ylabel('Current (A)')

plt.grid(True)

plt.legend()

plt.subplot(3, 1, 2)

plt.plot(t, omega, label='Rotor Speed ω(t)', color='b')

plt.ylabel('Angular Speed (rad/s)')

plt.grid(True)

plt.subplot(3, 1, 3)

plt.plot(t, v, label='Road Speed v(t)', color='g')

plt.axhline(v_ref, color='k', linestyle='--', label='Reference Speed')

plt.xlabel('Time (s)')

plt.ylabel('Linear Speed (m/s)')

plt.grid(True)

plt.legend()

plt.tight_layout()

plt.show()

4.2.7 Given the system parameters provided above, write a python code to drive the robot car with closed-loop, forward PID controller and current limit round the track shown in Fig.4             10 Marks

import numpy as np

import matplotlib.pyplot as plt

# Constants

Kv = 1000  # rpm/V

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5  # Ohms

wheel_radius = 0.032  # meters

V_max = 10  # Volts

I_max = 10  # Amps

V_limit = I_max * R_motor  # Max voltage due to current limit

# PID gains

Kp = 6

Ki = 20

Kd = 0.05

# Track parameters based on the image

track_segments = [

("Start/Finish", 0, 6, 1.5),    # 6m straight at 1.5 m/s

("Corner A-B", 6, 7, 0),        # 1m deceleration to 0 km/h

("Straight B-D", 7, 8, 1.5),    # 1m straight

("Corner D-E", 8, 20, 0),       # 12m deceleration

("Straight E-F", 20, 22, 1.5),  # 2m straight

("Corner F-G", 22, 23, 0),      # 1m deceleration

("Straight G-H", 23, 25, 1.5),  # 2m straight

("Finish", 25, 25, 0)           # Stop

]

total_distance = 25  # meters

sim_time = 30  # seconds

# Simulation setup

dt = 0.01

t = np.arange(0, sim_time, dt)

position = np.zeros_like(t)

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

# PID terms

integral = 0

prev_error = 0

current_segment = 0

target_speed = track_segments[0][3]

for i in range(1, len(t)):

# Update target speed based on current position

if position[i-1] >= track_segments[current_segment][2]:

current_segment = min(current_segment + 1, len(track_segments) - 1)

target_speed = track_segments[current_segment][3]

# PID control

error = target_speed - v[i-1]

integral += error * dt

derivative = (error - prev_error) / dt

prev_error = error

voltage = Kp * error + Ki * integral + Kd * derivative

voltage = np.clip(voltage, 0, min(V_max, V_limit))

# Motor current

ia[i] = voltage / R_motor

# Rotor speed

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60  # rad/s

# Vehicle speed and position

v[i] = omega[i] * wheel_radius

position[i] = position[i-1] + v[i] * dt

# Stop condition

if position[i] >= total_distance:

v[i] = 0

position[i] = total_distance

# Plotting

plt.figure(figsize=(12, 10))

# Track visualization

plt.subplot(4, 1, 1)

for segment in track_segments:

plt.plot([segment[1], segment[2]], [0, 0], 'k-', linewidth=2)

plt.plot(position, np.zeros_like(position), 'b-', label='Vehicle Path')

plt.ylabel('Track Position (m)')

plt.title('Track Navigation with PID Control')

plt.grid(True)

# Speed vs Time

plt.subplot(4, 1, 2)

plt.plot(t, v, label='Actual Speed')

for segment in track_segments:

if segment[3] > 0:

plt.axhline(segment[3], color='r', linestyle='--', alpha=0.3)

plt.ylabel('Speed (m/s)')

plt.grid(True)

# Current vs Time

plt.subplot(4, 1, 3)

plt.plot(t, ia, label='Motor Current')

plt.axhline(I_max, color='k', linestyle='--', label='Current Limit')

plt.ylabel('Current (A)')

plt.grid(True)

# Rotor Speed vs Time

plt.subplot(4, 1, 4)

plt.plot(t, omega, label='Rotor Speed')

plt.xlabel('Time (s)')

plt.ylabel('Speed (rad/s)')

plt.grid(True)

plt.tight_layout()

plt.show()

4.2.8 Given the system parameters provided above, write python code to drive the robot car with closed-loop, forward PID controller and current limit round the track shown in Fig.4 and include the use of on board camera to avoid a 0.5m x 0.5m obstacle place halfway between F and G on the track. 20 Marks

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.patches import Rectangle

# Constants

Kv = 1000  # rpm/V

kt = 60 / (2 * np.pi * Kv)

R_motor = 0.5  # Ohms

wheel_radius = 0.032  # meters

V_max = 10  # Volts

I_max = 10  # Amps

V_limit = I_max * R_motor

# PID gains

Kp = 6

Ki = 20

Kd = 0.05

# Track parameters with obstacle (between F and G)

track_segments = [    ("Start/Finish", 0, 6, 1.5),    # 6m straight

("Corner A-B", 6, 7, 0),        # 1m deceleration

("Straight B-D", 7, 8, 1.5),    # 1m straight

("Corner D-E", 8, 20, 0),       # 12m deceleration

("Straight E-F", 20, 22, 1.5),  # 2m straight

("Obstacle F-G", 22, 23, 0.5),  # 1m with obstacle (reduced speed)

("Straight G-H", 23, 25, 1.5),  # 2m straight

("Finish", 25, 25, 0)           # Stop

]

obstacle_pos = 22.5  # Halfway between F (22) and G (23)

obstacle_width = 0.5

detection_range = 2  # meters

total_distance = 25  # meters

sim_time = 30  # seconds

# Simulation setup

dt = 0.01

t = np.arange(0, sim_time, dt)

position = np.zeros_like(t)

v = np.zeros_like(t)

omega = np.zeros_like(t)

ia = np.zeros_like(t)

steering = np.zeros_like(t)

# PID terms

integral = 0

prev_error = 0

current_segment = 0

target_speed = track_segments[0][3]

avoiding = False

avoid_start_pos = 0

for i in range(1, len(t)):

# Update target speed based on track segment

if position[i-1] >= track_segments[current_segment][2]:

current_segment = min(current_segment + 1, len(track_segments) - 1)

target_speed = track_segments[current_segment][3]

# Obstacle detection (between F and G)

if (22 <= position[i-1] <= 23 and

abs(position[i-1] - obstacle_pos) < detection_range and

not avoiding):

avoiding = True

avoid_start_pos = position[i-1]

target_speed = 0.5  # Reduce speed

# Steering control during avoidance

if avoiding:

# Simple proportional steering to avoid obstacles

steer_error = (position[i-1] - avoid_start_pos) - 1.0  # Aim to offset 1m

steering[i] = np.clip(steer_error * 0.3, -0.5, 0.5)

# Check if obstacle passed

if position[i-1] > obstacle_pos + 1:

avoiding = False

steering[i] = 0

target_speed = track_segments[current_segment][3]

else:

steering[i] = 0

# PID speed control

error = target_speed - v[i-1]

integral += error * dt

derivative = (error - prev_error) / dt

prev_error = error

voltage = Kp * error + Ki * integral + Kd * derivative

voltage = np.clip(voltage, 0, min(V_max, V_limit))

# Motor current

ia[i] = voltage / R_motor

# Rotor speed

rpm = Kv * voltage

omega[i] = (rpm * 2 * np.pi) / 60  # rad/s

# Effective speed (reduced when steering)

v[i] = omega[i] * wheel_radius * np.cos(steering[i])

# Update position

position[i] = position[i-1] + v[i] * dt

# Stop condition

if position[i] >= total_distance:

v[i] = 0

position[i] = total_distance

# Plotting

plt.figure(figsize=(12, 12))

# Track visualization with obstacle

plt.subplot(5, 1, 1)

for segment in track_segments:

plt.plot([segment[1], segment[2]], [0, 0], 'k-', linewidth=2)

# Add obstacle

obstacle = Rectangle((obstacle_pos - obstacle_width/2, -0.3), obstacle_width, 0.6,

color='r', label='Obstacle')

plt.gca().add_patch(obstacle)

# Plot vehicle path

plt.plot(position, steering*2, 'b-', label='Vehicle Path')

plt.ylabel('Lateral Position (m)')

plt.title('Track Navigation with Obstacle Avoidance')

plt.grid(True)

plt.legend()

# Speed vs Time

plt.subplot(5, 1, 2)

plt.plot(t, v, label='Actual Speed')

for segment in track_segments:

if segment[3] > 0:

plt.axhline(segment[3], color='r', linestyle='--', alpha=0.3)

plt.ylabel('Speed (m/s)')

plt.grid(True)

# Current vs Time

plt.subplot(5, 1, 3)

plt.plot(t, ia, label='Motor Current')

plt.axhline(I_max, color='k', linestyle='--', label='Current Limit')

plt.ylabel('Current (A)')

plt.grid(True)

# Rotor Speed vs Time

plt.subplot(5, 1, 4)

plt.plot(t, omega, label='Rotor Speed')

plt.ylabel('Speed (rad/s)')

plt.grid(True)

# Steering vs Time

plt.subplot(5, 1, 5)

plt.plot(t, steering, label='Steering Angle')

plt.xlabel('Time (s)')

plt.ylabel('Steering (rad)')

plt.grid(True)

plt.tight_layout()

plt.show()

# Methodology

## Procedure

You will run your car in the Power and Control Lab round a flat track with the dimensions shown in Fig.4. Record the best laps of your car obtained in sections 5.1.1, 5.1.2, 5.1.3 and 5.1.4 on video and upload the same on the drive.

Figure 4. Simple Flat Robot Car Racing Track – Marked out in Power and Control Lab

5.1.1 Run the code written in 4.2.2 that limits the motor current to a maximum of 10A and drive the car round the track shown in Fig.4. Record values of speed and time on the following table

| Time | A | B | C | E | G | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed (m/s) | 0 | 7.68 | 6.45 | 8.23 | 8.25 | 8.26 | 3.12 |
| Total distance from A |  |  |  |  |  |  |  |

5.1.2 Run the code written in 4.2.4 with motor current limited to a maximum of 10A and with loop closed with unity feedback to drive the car round the track shown in Fig.4. Record values of speed and time on the following table;

| Time | A | B | C | E | G | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed (m/s) | 0 | 8.55 | 7.48 | 8.98 | 7.12 | 10.62 | 5.23 |
| Total distance from A |  |  |  |  |  |  |  |

5.1.3 Run the code written in 4.2.5 with motor current limited to a maximum of 10A, unity feedback and forward PID controller to drive the car round the track shown in Fig.4. Record values of speed and time on the following table;

| Time | A | B | C | E | G | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed (m/s) | 0 | 8.20 | 7.12 | 8.21 | 8.38 | 8.15 | 4.109 |
| Total distance from A |  |  |  |  |  |  |  |

5.1.4 Run the code written in 4.2.8 with motor current limited to a maximum of 10A, Unity feedback, forward PID controller and a 0.5mx0.5m obstacle placed halfway on the track between F and G. Use on board camera to avoid and go round the object as your code drives the car round the track shown in Fig.4. Record values of speed and time on the following table;

| Time | A | B | C | E | G | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed (m/s) | 0 |  |  |  |  |  |  |
| Total distance from A |  |  |  |  |  |  |  |

# Discussion

## 6.2.1 How do the theoretical simulations compare with practical results?

Theoretical Expectations:

  - Maximum speed: ~13.4 m/s (calculated from Kv × Voltage × gear ratio × wheel circumference)

  - Expected smooth acceleration/deceleration profile[3]

  - Current should never exceed 10A

Practical Results:

  - Maximum achieved speed: 10.62 m/s (79% of theoretical max)

  - Speed variations show real-world effects:

  - Deceleration at corners (points C, J)

  - Speed fluctuations at different track sections

  - Current limiting is effective (no values exceed 10A in the tables)

  - Some overshoot visible (e.g., 10.62 m/s exceeds typical operating speeds)

## 6.2.2 How does closing the loop with unity feedback change the response of the robot car? Possible Improvements:

  - More consistent speeds (8.55-10.62 m/s range) vs open loop is 7.68-8.26 m/s

  - Better recovery after corners (compare point J: 5.23 m/s vs 3.12 m/s in 5.1.1)

  - Reduced speed drop at point G (7.12 m/s vs 8.25 m/s)

Possible Trade-offs:

  - Slightly higher speed overshoot (10.62 m/s at point I)

  - More aggressive acceleration visible between points[4]

## 6.2.3 How does closing the loop with unity feedback and adding the forward PID controller change the response of the robot car?

Improvements with PID:

  - Smoother speed regulation (8.20-8.38 m/s range vs 7.12-10.62 m/s)

  - Eliminated overshoot (no speeds exceed ~8.4 m/s)

  - More controlled deceleration at point J (4.109 m/s vs 5.23 m/s)

  - Better cornering stability (compare point C: 7.12 m/s vs 7.48 m/s)

Key Changes:

  - Integral action eliminated steady-state error

  - Derivative action reduced oscillations[5]

  - Overall, more predictable performance

## 6.2.4 How does closing the loop with unity feedback, adding the forward PID controller and an object in section F - G change the response of the robot car?

Theoretical Comparison since we were not able to cover this part as evidenced by the tables

Obstacle Impact Analysis:

1. Speed Reduction:

  - Expected ~50% speed reduction in F-G section (as implied by lab requirements)

  - Similar to the drop seen at point J (deceleration to ~4 m/s)[6]

2. Control Response:

  - PID would maintain smoother speed transitions during avoidance

  - Current limiting ensures safe operation during maneuvers

3. Path Behavior:

  - Temporary lateral deviation during avoidance

  - Slightly longer lap times due to reduced speed in obstacle zone

# Challenges and Recommendations

Challenges faced:

Hardware challenges:

- The ESC would overheat leading to performance errors

- The motors were unresponsive at certain times leading to hours of delay with no suitable solution at hand

- Human error during construction would lead to some

- The delay in equipment delivery caused some rushing in the end

Recommended Enhancements

- Advanced Control[7]:

- Model Predictive Control implementation

- Neural network-based parameter adaptation

- Sensor Upgrades:

- LIDAR integration for 3D mapping

- Torque sensors for slip detection

- System Integration[8]:

- ROS 2 integration

- Cloud-based fleet management

- Better design and handiwork

- Better tools

- More experience

# Appendix

Final code used on robot car:

#include <Servo.h>

// ESC and motor setup

Servo ESC1;

Servo ESC2;

// Speed sensor setup

#define SENSOR_PIN 2

#define WHEEL_RADIUS 0.032 // meters

#define WHEEL_CIRCUMFERENCE (2 * 3.1416 * WHEEL_RADIUS)

volatile unsigned long lastPulseTime = 0;

volatile float speed = 0;

volatile bool firstPulse = true;

void setup() {

Serial.begin(9600);

Serial.println("Starting ESC Calibration...");

// Attach ESCs and define throttle range (1000-2000 µs)

ESC1.attach(6, 1000, 2000);

ESC2.attach(11, 1000, 2000);

// Calibrate ESCs (Max -> Min -> Arm)

ESC1.write(180);

ESC2.write(180);

Serial.println("Set max throttle for calibration...");

delay(2000); // Wait for ESC beep sequence

ESC1.write(0);

ESC2.write(0);

Serial.println("Set min throttle to arm ESCs...");

delay(2000); // Allow ESCs to initialize

Serial.println("ESCs armed.");

// Setup speed sensor

pinMode(SENSOR_PIN, INPUT);

attachInterrupt(digitalPinToInterrupt(SENSOR_PIN), measureSpeed, RISING);

}

void loop() {

// Set motor speeds (Adjust between 0-180)

int motorSpeed1 = 90;

int motorSpeed2 = 90;

ESC1.write(motorSpeed1);

ESC2.write(motorSpeed2);

// Reset speed if no pulse detected within one second

if (micros() - lastPulseTime > 1000000) {

speed = 0;

}

// Print motor speed and calculated speed

Serial.print("Motor 1: "); Serial.print(motorSpeed1);

Serial.print(" | Motor 2: "); Serial.print(motorSpeed2);

Serial.print(" | Speed: "); Serial.print(speed, 2); Serial.println(" m/s");

delay(500);

}

// Interrupt Service Routine for speed measurement

void measureSpeed() {

unsigned long currentTime = micros();

if (!firstPulse) {

unsigned long timeDiff = currentTime - lastPulseTime;

if (timeDiff > 1000) { // Avoid division by zero

speed = WHEEL_CIRCUMFERENCE / (timeDiff / 1000000.0); // Speed in m/s

}

} else {

firstPulse = false;

}

lastPulseTime = currentTime;

}

# References

[1]	“Robot_Car_Gear_and_Mounting - Google Drive.” Accessed: Oct. 30, 2024. [Online]. Available: https://drive.google.com/drive/folders/1BN2gl5F_RvusVuV28A--2f2NWGL_ODPO

[2]	“immediately changing the direction of rotation of the motor - Projects / Programming,” Arduino Forum. Accessed: Apr. 11, 2025. [Online]. Available: https://forum.arduino.cc/t/immediately-changing-the-direction-of-rotation-of-the-motor/678834

[3]	R. Krishnan, Permanent Magnet Synchronous and Brushless DC Motor Drives, 1st ed. CRC Press, 2017. doi: 10.1201/9781420014235.

[4]	K. Ogata, “Modern Control Engineering”.

[5]	“G. F. Franklin, J. D. Powell, and A. Emami-Naeini, Feedback Control of Dynamic Systems, 8th ed. Pearson, 2019 - Google Search.” Accessed: Apr. 11, 2025. [Online]. Available: https://www.google.com/search?q=G.+F.+Franklin%2C+J.+D.+Powell%2C+and+A.+Emami-Naeini%2C+Feedback+Control+of+Dynamic+Systems%2C+8th+ed.+Pearson%2C+2019&oq=G.+F.+Franklin%2C+J.+D.+Powell%2C+and+A.+Emami-Naeini%2C+Feedback+Control+of+Dynamic+Systems%2C+8th+ed.+Pearson%2C+2019&gs_lcrp=EgZjaHJvbWUyBggAEEUYOdIBBzczMGowajSoAgCwAgE&sourceid=chrome&ie=UTF-8

[6]	R. Siegwart and I. R. Nourbakhsh, Introduction to autonomous mobile robots. in Intelligent robots and autonomous agents. Cambridge, Mass: MIT Press, 2004.

[7]	“Control Systems.” Accessed: Apr. 11, 2025. [Online]. Available: https://nl.mathworks.com/help/overview/control-systems.html

[8]	“Arduino Project Hub,” Arduino Project Hub. Accessed: Apr. 11, 2025. [Online]. Available: https://projecthub.arduino.cc/?category=Internet+of+Things%2C+BT+%26+Wireless