SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONICS ENGINEERING (BSEEE)

BEE 3202: TRANSMISSION LINES AND WAVEGUIDES

LAB 2:

MEASUREMENT OF INCIDENT AND REFLECTED WAVES IN A TRANSMISSION LINE

BY:

Group 2:

Esther Wangari 					145822

Mwongela Edward Aribu 				142092

Trevor Ayimba					137358

William Munene					145590

Lecturer: Dr. Shamim Omar Nassar

Contents

List of figures	2

1. Aim	3

2. Equipment	3

3. Procedure	4

4. Observations	5

5. Analysis	6

6. Conclusion	7

Appendices	8

## List of figures

No table of figures entries found.

To measure and analyse the incident (Vᵢ) and reflected (Vᵣ) waves in a microstrip transmission line terminated with an open load and plot the Distance vs. Power graph to study standing wave behaviour.

# 2. Equipment

| Device/Accessory | Model/Type | Quantity |
| --- | --- | --- |
| Microstrip Transmission Line | Transmission Line JIG | 1 |
| RFL-MMTLT Control Unit | Control Unit | 1 |
| SMA Cables | 2-meter & 1-meter (50 Ω) | 3 |
| Termination Load | OPEN | 1 |
| Software | Transmission Line Software | 1 |

# 3. Procedure

- Setup Connections:

  - Connect the transmission line to the RFL-MMTLT control unit using SMA cables.

  - Terminate Port 3 of the transmission line with an OPEN load.

- Software Configuration:

  - Open the Transmission Line Software.

  - Navigate to:

Operating Mode → Transmission Line → Standing Wave → Incident (Vᵢ) & Reflected (Vᵣ) Waves

  - Set Resolution = 5 mm and Frequency = 275 MHz.

- Data Acquisition:

  - Click RUN to initiate the sensor probe.

  - The probe traverses the line, recording power levels at intervals.

- Graph Interpretation:

  - Yellow Plot: Incident wave (Vᵢ).

  - Red Plot: Reflected wave (Vᵣ).

# 4. Observations

- Reflected Power: ~6 dBm (due to open termination, 100% reflection expected).

- Incident Power Attenuation:

  - 16 dBm at 50 mm → 5 dBm at 550 mm.

- Standing Wave Formation:

  - Peaks and nulls observed due to constructive/destructive interference.

# 5. Analysis

5.1 Theoretical Expectations

- Open Termination:

  - Reflection coefficient (Γ) = +1 → Total reflection.

  - Standing Wave Ratio (SWR) = ∞.

- Power Loss:

  - Due to conductor/dielectric losses in the microstrip line.

5.2 Comparison with Simulation

| Parameter | Theoretical | Observed | Deviation |
| --- | --- | --- | --- |
| Reflected Power | 6 dBm | ~6 dBm | Minimal |
| Incident Power Drop | Exponential | Linear trend | Cable losses? |

# 6. Conclusion

- Open Termination Behaviour: Confirmed 100% reflection (~6 dBm reflected power).

- Attenuation: Incident power decreased linearly due to radiation losses and cable imperfections.

- Practical Insight: Open-terminated lines are unsuitable for power transfer; matched loads preferred.

Recommendations:

- Repeat with short-circuit and matched (50 Ω) terminations.

- Use higher-resolution probes (e.g., 1 mm) for precise null detection.

# Appendices

- Raw Data Table:

| Distance (mm) | Incident Power (dBm) | Reflected Power (dBm) |
| --- | --- | --- |
| 50 | 16 | 6 |
| ... | ... | ... |
| 550 | 5 | 6 |

MATLAB Code for Plotting:

distance = [50, ..., 550];

Vi = [16, ..., 5]; Vr = [6, ..., 6];

plot(distance, Vi, 'y', distance, Vr, 'r');

xlabel('Distance (mm)'); ylabel('Power (dBm)');

legend('Incident', 'Reflected');