SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONICS

ENGINEERING (BSEEE)

DIGITAL CONTROL 1

Lab 2:

AUTONOMOUS FLYING ROBOT CAR (DRONE)

BY

Edward Mwongela – 142092

Group members

Harry Boto

Arnold Injeni

Samuel Mwichigi

Trevor Ayimba

Lecturer: Prof. Jackson Mbuthia

30th March 2025

Contents

List of figures	3

Objective:	4

Learning Outcome:	4

Introduction	5

Components Required	5

1.	Flying Robot Car (Drone) Assembly	5

2.	Experiment Design – Write Code to Fly the Robot Car	6

Robot car - BLDC Motor	6

Flying Robot Car (Drone) Symbols	7

Coordinate System	7

Drone Dynamics	8

Translational Motion (Newton’s Second Law)	8

Equation (1) in state space form	8

Motor Dynamics	9

Equation (2) in state space form	9

Rotational Motion (Euler’s Equations)	11

equation (3) in state space	11

State Space Equations for Drone Control in the complete state form	12

Drone Simulations	15

4.5 Write python code to simulate the drawn lift to 2m, hover, turn right, move forward 2m, turn left, move forward 2m, reverse 2m and softly land. Modify model parameters as necessary to achieve a good and stable response. Present your simulation graphs.	15

4.6 Add closed loop and PID control to the x, y, and z movements and repeat the movements described in 4.5. Present your simulation graphs             10 Marks	26

4.7 Use the code written in 4.6 repeating the movement described 4.5 and use on board camera to make the drawn land at a designated square of 0.1m by 0.1 10 Marks	37

Discussion	54

6.2.1 How do the theoretical simulations compare with practical results?	54

6.2.2 How does closing the loop with and adding the forward PID controller change the response of the flying robot car?	55

References	56

## List of figures

Figure 1. Brushless DC Motor – 3 Phase Star Connected Armature (Stator or Rotor)	5

Figure 2. Electronic Speed Controller	5

Figure 3. Motor, Raspberry Pi, ESC and Battery Mounting	6

Figure 4. Battery  Clamp and Motor Mount	6

Figure 5. position, attitude, angular velocity and velocity versus time	25

Figure 6. motor voltages versus time and 3D trajectory	26

Figure 7. position, attitude, motor voltage and velocity versus time	36

Figure 8. 3D trajectory and position errors	37

# Objective:

- Lift, hover, turn left, turn right and land the autonomous flying robot car [1]

- Implement control laws to smoothly fly the robot car

- Land the flying robot car at a designated square

# Learning Outcome:

- Apply code running on a Raspberry PI to autonomously fly the robot car

- Develop control algorithms to create smooth flight for the flying robot car

- Apply camera vision to smoothly land the drone at a designated square.

# Introduction

## Components Required

- Microcontroller (e.g., Raspberry PI)

- Four BLDC motors with Four drivers (e.g., ESCs)

- Power supply (LiPo or Lithium-Ion battery)

- Sensors (e.g., on-board camera for landing)

- Computer for code development

- Robot car assembled in Lab1

## Flying Robot Car (Drone) Assembly

The flying robot car uses four brushless motors shown in Fig. 1 to drive two front propellers and two rear propellers as shown in Fig. 3. Four Electronic Speed Controllers (ESC) shown in Fig. 2 drive propellers P1, P2, P3 and P4. The robot car assembly will fly complete with the wheels and your bearing mounts. To fly the robot car, securely mount the wheels, gears, motors, battery, Raspberry PI, ESCs as shown in Fig. 3. Tether your flying robot car with a strong string to limit maximum height and forward/backward travel so that you can pull it down as you evaluate your autonomous flight program.

Figure 1. Brushless DC Motor – 3 Phase Star Connected Armature (Stator or Rotor)

Figure 2. Electronic Speed Controller

Figure 3. Motor, Raspberry Pi, ESC and Battery Mounting

Figure 4. Battery  Clamp and Motor Mount

## Experiment Design – Write Code to Fly the Robot Car

### Robot car - BLDC Motor

### Flying Robot Car (Drone) Symbols

M - drone mass = 2kg

g - gravity acceleration = 9.81m/s22

l - drone motor mount arm = 0.2m

J_x, J_y, J_z - moment of inertia on each axis = mI22/12 = 0.006kgm22

Β - drag coefficient along x, y, z axis = 5x10−8−8 Nms22/rad22

b - propeller motion damping coefficient = 0.0009Ns/rad

kq - motor torque coefficient (depends on propeller size and air resistance) = 2.5x10^−6Ns^2/rad^2

kb - motor emf constant = 0.01 Vs/rad

kt - motor torque constant = 0.01Nm/A

ω_r - rotor angular speed rad/s

ω_x, ω_y, ω_z - drone angular speed rad/s in x, y, z axis

R_m - motor winding resistance = 0.09Ω

L_m - motor winding inductance = 0.05H

J_m - motor/propeller assembly moment of inertia = 0.002kgm22

Vm1, Vm2, Vm3, Vm4 - motor input voltage = 12V maximum

fm1, fm2, fm3, fm4 - motor current = 10A maximum km motor thrust force constant 0.08 N/V

F1, F2, F3, F4 - force/thrust developed by each motor (N)

Q1, Q2, Q3, Q4 - reaction torque caused by aerodynamic drag from the spinning propeller

### Coordinate System

X,Y,Z - describes drone's absolute position

x˙velocity - in x direction

y˙velocity - in y direction

z˙ - vertical velocity

#### Body Frame {x,y,z}{x,y,z}

Moves with the drone

Forces and torques applied in this frame

angles (ϕ,θ,ψϕ,θ,ψ) describe drone orientation

#### Motor Rotation Directions

CW Motors: 1, 3

CCW Motors: 2, 4

Each motor produces thrust F1, F2, F3, F4

## Drone Dynamics

### Translational Motion (Newton’s Second Law)

m[x¨,y¨,z¨ T = ∑ F

Forces acting on the drone

Gravity Fg

Aerodynamic drag Fp

Thrust force from motors F1, F2, F3, F4

## Equation (1) in state space form

State Variables:

the state vector X as:

State-Space Representation:

The state-space form is X˙=AX+Bu, where u is the input (motor thrusts)

State Derivatives:

- Position derivatives:

Velocity derivatives (from given equation):

=

Where:

- k is the thrust coefficient (relating motor input to force)

- The gravity term is treated as a constant disturbance

- The drag coefficient β appears in the A matrix

- Motor thrusts F1F1​ to F4F4​ are the control inputs

### Motor Dynamics

For each motor

di(t)dt=−i(t)RmLm−kbLmω1(t)+vm(t)Lmdt

dω1(t)dt=−QiJm+kfJmi(t)dt

Qi=kdωi2

## Equation (2) in state space form

Where:

- Q1​ = Reaction torque from motor 1 (Nm)

- kq​ = Motor torque coefficient (Ns²/rad²)

- ω1​ = Angular velocity of motor 1 (rad/s)

1. Motor Dynamics Context:

The reaction torque Qi appears in the motor's rotational dynamics:

where:

- Jm​ = Motor+propeller inertia

- kt​ = Torque constant

- i_i​ = Motor current

- b = Damping coefficient

2. State Variables:

For a single motor, define the state vector as:

3. State-Space Equations:

Using the motor electrical and mechanical equations:

4. Nonlinear State-Space Form:

5. Linearized Form (for small perturbations):

At operating point (ω0,i0):

Where:

- ω~i=ωi−ω0

- i~i=ii−i0

- V~i=Vi−V0

### Rotational Motion (Euler’s Equations)

jω˙=τ−Jω2

J=[Jx,Jy,Jz]

ω=[ωx,ωy,ωz]

τ=[τ0,τφ,τψ]are torques

τ0=I[F2−F4]τφ=Q1−Q2+Q3−Q4 ​

equation (3) in state space

jω˙=τ−Jω2

Where:

- J = Moment of inertia tensor (diagonal matrix with Jx,Jy,Jz ​)

- ω = Angular velocity vector [ωx,ωy,ωz]T

- τ = Torque vector [τϕ,τθ,τψ]T

1.Rewrite the Original Equation isolating ω˙

2.Define State Variables

3.State-Space Representation

The state-space form is X˙=f(X,τ).

Substituting the given equation

4.Linearize around an equilibrium point (e.g., hover condition ω=0):

X˙≈J−1τ

This yields a linear state-space form:

Final State-Space Form:

where X2=[ωx2,ωy2,ωz2​]T, and J−1=diag(1/Jx,1/Jy,1/Jz).

## State Space Equations for Drone Control in the complete state form

10 Marks

- Given:

m[x¨,y¨,z¨]T=∑F

State Variables:

Xtrans=[x,y,z,x˙,y˙,z˙]T

State-Space Equations:

Matrix Form:

X˙trans​=

where u_motors=[F1,F2,F3,F4]^T

- Given:

Qi=kqωi2

​Jmω˙i=ktii−Qi−bωi

State Variables:

Xmotor=[ω1,ω2,ω3,ω4,i1,i2,i3,i4]^T

State-Space Equations:

Matrix Form (Nonlinear):

where Vinput=[V1,V2,V3,V4]^T

- Given:

Jω˙=τ−Jω^2

State Variables:

Xrot=[ϕ,θ,ψ,ωx,ωy,ωz]T

State-Space Equations:

Matrix Form (Nonlinear):

Input Coupling:
Torques τ are generated by motor thrusts:

where l is the arm length, and Qi=kqωi2

- Complete Unified State-Space Model

Combine all subsystems into a single state vector:

Final Form:

X˙=f(X,u)

where:

- u=[V1,V2,V3,V4] T (motor voltages).

- f(⋅) integrates all nonlinear dynamics above.

# Drone Simulations

## 4.5 Write python code to simulate the drawn lift to 2m, hover, turn right, move forward 2m, turn left, move forward 2m, reverse 2m and softly land. Modify model parameters as necessary to achieve a good and stable response. Present your simulation graphs.

10 Marks

import numpy as np

import matplotlib.pyplot as plt

from mpl_toolkits.mplot3d import Axes3D

class DroneSimulator:

def __init__(self):

# Drone parameters

self.mass = 2.0       # kg

self.g = 9.81         # m/s²

self.kf = 0.08        # Thrust coefficient (N/V)

self.km = 0.01        # Torque coefficient (Nm/A)

self.arm_length = 0.2 # m

# Initial state [x, y, z, vx, vy, vz, phi, theta, psi, p, q, r]

self.state = np.zeros(12)

# Control limits

self.max_voltage = 12.0  # V

self.min_voltage = 0.0   # V

# Simulation parameters

self.dt = 0.01       # Time step

self.time = 0         # Current time

def dynamics(self, state, t, motor_voltages):

"""Drone dynamics model"""

x, y, z, vx, vy, vz, phi, theta, psi, p, q, r = state

V1, V2, V3, V4 = motor_voltages

# Calculate forces and torques

F1 = self.kf * V1

F2 = self.kf * V2

F3 = self.kf * V3

F4 = self.kf * V4

# Total thrust (in body frame)

thrust = F1 + F2 + F3 + F4

# Torques (roll, pitch, yaw)

tau_phi = self.arm_length * (F2 - F4)

tau_theta = self.arm_length * (F3 - F1)

tau_psi = self.km * (F1 - F2 + F3 - F4)

# Rotation matrix (body to world)

R = np.array([

[np.cos(theta)*np.cos(psi),

np.sin(phi)*np.sin(theta)*np.cos(psi)-np.cos(phi)*np.sin(psi),

np.cos(phi)*np.sin(theta)*np.cos(psi)+np.sin(phi)*np.sin(psi)],

[np.cos(theta)*np.sin(psi),

np.sin(phi)*np.sin(theta)*np.sin(psi)+np.cos(phi)*np.cos(psi),

np.cos(phi)*np.sin(theta)*np.sin(psi)-np.sin(phi)*np.cos(psi)],

[-np.sin(theta),

np.sin(phi)*np.cos(theta),

np.cos(phi)*np.cos(theta)]

])

# Translational dynamics

thrust_vector = np.array([0, 0, thrust])

gravity_vector = np.array([0, 0, -self.mass*self.g])

acceleration = (R @ thrust_vector + gravity_vector) / self.mass

# Rotational dynamics

inertia = np.diag([0.01, 0.01, 0.02])  # kg·m²

omega = np.array([p, q, r])

tau = np.array([tau_phi, tau_theta, tau_psi])

omega_dot = np.linalg.inv(inertia) @ (tau - np.cross(omega, inertia @ omega))

# Kinematics

euler_rates = np.array([

p + q*np.sin(phi)*np.tan(theta) + r*np.cos(phi)*np.tan(theta),

q*np.cos(phi) - r*np.sin(phi),

(q*np.sin(phi) + r*np.cos(phi)) / np.cos(theta)

])

return np.concatenate((

[vx, vy, vz],

acceleration,

euler_rates,

omega_dot

))

def mission_control(self, t):

"""Mission sequence controller"""

if t < 5:

# Phase 1: Lift to 2m

target_z = 2.0 * (t/5)

return self.altitude_controller(target_z)

elif t < 10:

# Phase 2: Hover at 2m

return self.altitude_controller(2.0)

elif t < 15:

# Phase 3: Turn right (90 degrees)

target_psi = np.pi/2 * ((t-10)/5)

return self.attitude_controller(0, 0, target_psi, 2.0)

elif t < 20:

# Phase 4: Move forward 2m

target_x = 2.0 * ((t-15)/5)

return self.position_controller(target_x, 0, 2.0, 0, 0, np.pi/2)

elif t < 25:

# Phase 5: Turn left (90 degrees)

target_psi = np.pi/2 - np.pi/2 * ((t-20)/5)

return self.attitude_controller(0, 0, target_psi, 2.0)

elif t < 30:

# Phase 6: Move forward 2m

target_x = 2.0 + 2.0 * ((t-25)/5)

return self.position_controller(target_x, 0, 2.0, 0, 0, 0)

elif t < 35:

# Phase 7: Reverse 2m

target_x = 4.0 - 2.0 * ((t-30)/5)

return self.position_controller(target_x, 0, 2.0, 0, 0, 0)

else:

# Phase 8: Soft landing

landing_time = 40 - t  # 5 seconds for landing

target_z = 2.0 * (landing_time/5) if landing_time > 0 else 0

return self.altitude_controller(target_z)

def altitude_controller(self, target_z):

"""Simple altitude controller"""

error_z = target_z - self.state[2]

base_thrust = self.mass * self.g / (4 * self.kf)

adjustment = error_z * 0.5

# Equal thrust to all motors

V = np.clip(base_thrust + adjustment, self.min_voltage, self.max_voltage)

return np.array([V, V, V, V])

def attitude_controller(self, target_phi, target_theta, target_psi, target_z):

"""Attitude controller with altitude hold"""

# Altitude control

error_z = target_z - self.state[2]

base_thrust = self.mass * self.g / (4 * self.kf)

thrust_adjustment = error_z * 0.5

# Attitude control

error_phi = target_phi - self.state[6]

error_theta = target_theta - self.state[7]

error_psi = target_psi - self.state[8]

roll_adjust = error_phi * 0.8

pitch_adjust = error_theta * 0.8

yaw_adjust = error_psi * 0.5

# Motor mixing

V1 = base_thrust + thrust_adjustment - roll_adjust + pitch_adjust + yaw_adjust

V2 = base_thrust + thrust_adjustment + roll_adjust + pitch_adjust - yaw_adjust

V3 = base_thrust + thrust_adjustment + roll_adjust - pitch_adjust + yaw_adjust

V4 = base_thrust + thrust_adjustment - roll_adjust - pitch_adjust - yaw_adjust

return np.clip([V1, V2, V3, V4], self.min_voltage, self.max_voltage)

def position_controller(self, target_x, target_y, target_z, target_phi, target_theta, target_psi):

"""Position controller with attitude control"""

# Position errors

error_x = target_x - self.state[0]

error_y = target_y - self.state[1]

error_z = target_z - self.state[2]

# Convert to attitude targets

target_theta = error_x * 0.2

target_phi = -error_y * 0.2

return self.attitude_controller(target_phi, target_theta, target_psi, target_z)

def simulate(self, total_time=40):

"""Run the complete simulation"""

time_points = np.arange(0, total_time, self.dt)

states = []

controls = []

for t in time_points:

# Get control inputs

motor_voltages = self.mission_control(t)

# Integrate dynamics

state_dot = self.dynamics(self.state, t, motor_voltages)

self.state += state_dot * self.dt

# Store results

states.append(self.state.copy())

controls.append(motor_voltages.copy())

return time_points, np.array(states), np.array(controls)

def plot_results(self, time, states, controls):

"""Plot simulation results"""

plt.figure(figsize=(15, 10))

# Position plot

plt.subplot(3, 2, 1)

plt.plot(time, states[:, 0], label='X')

plt.plot(time, states[:, 1], label='Y')

plt.plot(time, states[:, 2], label='Z')

plt.title('Position vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Position (m)')

plt.legend()

plt.grid()

# Attitude plot

plt.subplot(3, 2, 2)

plt.plot(time, np.degrees(states[:, 6]), label='Roll (φ)')

plt.plot(time, np.degrees(states[:, 7]), label='Pitch (θ)')

plt.plot(time, np.degrees(states[:, 8]), label='Yaw (ψ)')

plt.title('Attitude vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Angle (deg)')

plt.legend()

plt.grid()

# Velocity plot

plt.subplot(3, 2, 3)

plt.plot(time, states[:, 3], label='Vx')

plt.plot(time, states[:, 4], label='Vy')

plt.plot(time, states[:, 5], label='Vz')

plt.title('Velocity vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Velocity (m/s)')

plt.legend()

plt.grid()

# Angular velocity plot

plt.subplot(3, 2, 4)

plt.plot(time, np.degrees(states[:, 9]), label='p')

plt.plot(time, np.degrees(states[:, 10]), label='q')

plt.plot(time, np.degrees(states[:, 11]), label='r')

plt.title('Angular Velocity vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Angular velocity (deg/s)')

plt.legend()

plt.grid()

# Control inputs

plt.subplot(3, 2, 5)

plt.plot(time, controls[:, 0], label='Motor 1')

plt.plot(time, controls[:, 1], label='Motor 2')

plt.plot(time, controls[:, 2], label='Motor 3')

plt.plot(time, controls[:, 3], label='Motor 4')

plt.title('Motor Voltages vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Voltage (V)')

plt.legend()

plt.grid()

# 3D trajectory

plt.subplot(3, 2, 6, projection='3d')

plt.plot(states[:, 0], states[:, 1], states[:, 2])

plt.title('3D Trajectory')

plt.xlabel('X (m)')

plt.ylabel('Y (m)')

plt.gca().set_zlabel('Z (m)')

# Mark mission phases

for t in [5, 10, 15, 20, 25, 30, 35]:

idx = int(t / self.dt)

plt.gca().scatter(states[idx, 0], states[idx, 1], states[idx, 2],

color='red', s=50, label=f'{t}s' if t == 5 else None)

plt.legend()

plt.tight_layout()

plt.show()

# Run simulation

simulator = DroneSimulator()

time, states, controls = simulator.simulate()

simulator.plot_results(time, states, controls)

Figure 5. position, attitude, angular velocity, and velocity versus time

Figure 6. motor voltages versus time and 3D trajectory

## 4.6 Add closed loop and PID control to the x, y, and z movements and repeat the movements described in 4.5. Present your simulation graphs             10 Marks

import numpy as np

import matplotlib.pyplot as plt

from scipy.integrate import odeint

from mpl_toolkits.mplot3d import Axes3D

class PIDController:

def __init__(self, kp, ki, kd, limit):

self.kp = kp

self.ki = ki

self.kd = kd

self.limit = limit

self.integral = 0

self.prev_error = 0

def update(self, error, dt):

self.integral += error * dt

derivative = (error - self.prev_error) / dt

output = self.kp * error + self.ki * self.integral + self.kd * derivative

self.prev_error = error

return np.clip(output, -self.limit, self.limit)

class DroneSimulator:

def __init__(self):

# Drone parameters

self.mass = 2.0       # kg

self.g = 9.81         # m/s²

self.kf = 0.08        # Thrust coefficient (N/V)

self.km = 0.01        # Torque coefficient (Nm/A)

self.arm_length = 0.2 # m

self.inertia = np.diag([0.01, 0.01, 0.02])  # kg·m²

# PID controllers

self.x_pid = PIDController(1.2, 0.05, 0.8, 2.0)

self.y_pid = PIDController(1.2, 0.05, 0.8, 2.0)

self.z_pid = PIDController(2.0, 0.1, 1.0, 3.0)

self.phi_pid = PIDController(0.8, 0.02, 0.4, 1.0)

self.theta_pid = PIDController(0.8, 0.02, 0.4, 1.0)

self.psi_pid = PIDController(0.5, 0.01, 0.3, 1.0)

# Initial state [x, y, z, vx, vy, vz, phi, theta, psi, p, q, r]

self.state = np.zeros(12)

self.time = 0

def dynamics(self, state, t, motor_voltages):

"""Drone dynamics model"""

x, y, z, vx, vy, vz, phi, theta, psi, p, q, r = state

V1, V2, V3, V4 = motor_voltages

# Calculate forces and torques

F1 = self.kf * V1

F2 = self.kf * V2

F3 = self.kf * V3

F4 = self.kf * V4

# Total thrust (in body frame)

thrust = F1 + F2 + F3 + F4

# Torques (roll, pitch, yaw)

tau_phi = self.arm_length * (F2 - F4)

tau_theta = self.arm_length * (F3 - F1)

tau_psi = self.km * (F1 - F2 + F3 - F4)

# Rotation matrix (body to world)

R = np.array([

[np.cos(theta)*np.cos(psi),

np.sin(phi)*np.sin(theta)*np.cos(psi)-np.cos(phi)*np.sin(psi),

np.cos(phi)*np.sin(theta)*np.cos(psi)+np.sin(phi)*np.sin(psi)],

[np.cos(theta)*np.sin(psi),

np.sin(phi)*np.sin(theta)*np.sin(psi)+np.cos(phi)*np.cos(psi),

np.cos(phi)*np.sin(theta)*np.sin(psi)-np.sin(phi)*np.cos(psi)],

[-np.sin(theta),

np.sin(phi)*np.cos(theta),

np.cos(phi)*np.cos(theta)]

])

# Translational dynamics

thrust_vector = np.array([0, 0, thrust])

gravity_vector = np.array([0, 0, -self.mass*self.g])

acceleration = (R @ thrust_vector + gravity_vector) / self.mass

# Rotational dynamics

omega = np.array([p, q, r])

tau = np.array([tau_phi, tau_theta, tau_psi])

omega_dot = np.linalg.inv(self.inertia) @ (tau - np.cross(omega, self.inertia @ omega))

# Kinematics

euler_rates = np.array([

p + q*np.sin(phi)*np.tan(theta) + r*np.cos(phi)*np.tan(theta),

q*np.cos(phi) - r*np.sin(phi),

(q*np.sin(phi) + r*np.cos(phi)) / np.cos(theta)

])

return np.concatenate((

[vx, vy, vz],

acceleration,

euler_rates,

omega_dot

))

def mission_control(self, t):

"""Mission sequence with PID control"""

if t < 5:

# Phase 1: Lift to 2m

target = [0, 0, min(2.0, 2.0*(t/5)), 0, 0, 0]

elif t < 10:

# Phase 2: Hover at 2m

target = [0, 0, 2.0, 0, 0, 0]

elif t < 15:

# Phase 3: Turn right (90 degrees)

target = [0, 0, 2.0, 0, 0, np.pi/2 * ((t-10)/5)]

elif t < 20:

# Phase 4: Move forward 2m

target = [2.0 * ((t-15)/5), 0, 2.0, 0, 0, np.pi/2]

elif t < 25:

# Phase 5: Turn left (90 degrees)

target = [2.0, 0, 2.0, 0, 0, np.pi/2 - np.pi/2 * ((t-20)/5)]

elif t < 30:

# Phase 6: Move forward 2m

target = [2.0 + 2.0 * ((t-25)/5), 0, 2.0, 0, 0, 0]

elif t < 35:

# Phase 7: Reverse 2m

target = [4.0 - 2.0 * ((t-30)/5), 0, 2.0, 0, 0, 0]

else:

# Phase 8: Soft landing

landing_time = max(0, 40 - t)

target = [2.0, 0, 2.0 * (landing_time/5), 0, 0, 0]

return self.pid_controller(target)

def pid_controller(self, target):

"""Full PID control for all axes"""

x_target, y_target, z_target, phi_target, theta_target, psi_target = target

x, y, z, _, _, _, phi, theta, psi, _, _, _ = self.state

# Position control

x_error = x_target - x

y_error = y_target - y

z_error = z_target - z

# Convert position errors to attitude targets

theta_target = self.x_pid.update(x_error, self.dt)

phi_target = -self.y_pid.update(y_error, self.dt)

thrust_adjust = self.z_pid.update(z_error, self.dt)

# Attitude control

phi_error = phi_target - phi

theta_error = theta_target - theta

psi_error = psi_target - psi

roll_adjust = self.phi_pid.update(phi_error, self.dt)

pitch_adjust = self.theta_pid.update(theta_error, self.dt)

yaw_adjust = self.psi_pid.update(psi_error, self.dt)

# Motor mixing

base_thrust = self.mass * self.g / (4 * self.kf)

V1 = base_thrust + thrust_adjust - roll_adjust + pitch_adjust + yaw_adjust

V2 = base_thrust + thrust_adjust + roll_adjust + pitch_adjust - yaw_adjust

V3 = base_thrust + thrust_adjust + roll_adjust - pitch_adjust + yaw_adjust

V4 = base_thrust + thrust_adjust - roll_adjust - pitch_adjust - yaw_adjust

return np.clip([V1, V2, V3, V4], 0, 12.0)

def simulate(self, dt=0.01, total_time=40):

"""Run the complete simulation"""

self.dt = dt

time_points = np.arange(0, total_time, dt)

states = []

controls = []

for t in time_points:

motor_voltages = self.mission_control(t)

state_dot = self.dynamics(self.state, t, motor_voltages)

self.state += state_dot * dt

states.append(self.state.copy())

controls.append(motor_voltages.copy())

return time_points, np.array(states), np.array(controls)

def plot_results(self, time, states, controls):

"""Plot simulation results"""

plt.figure(figsize=(18, 12))

# Position plot

plt.subplot(3, 2, 1)

plt.plot(time, states[:, 0], label='X')

plt.plot(time, states[:, 1], label='Y')

plt.plot(time, states[:, 2], label='Z')

plt.title('Position vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Position (m)')

plt.legend()

plt.grid()

# Attitude plot

plt.subplot(3, 2, 2)

plt.plot(time, np.degrees(states[:, 6]), label='Roll (φ)')

plt.plot(time, np.degrees(states[:, 7]), label='Pitch (θ)')

plt.plot(time, np.degrees(states[:, 8]), label='Yaw (ψ)')

plt.title('Attitude vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Angle (deg)')

plt.legend()

plt.grid()

# Velocity plot

plt.subplot(3, 2, 3)

plt.plot(time, states[:, 3], label='Vx')

plt.plot(time, states[:, 4], label='Vy')

plt.plot(time, states[:, 5], label='Vz')

plt.title('Velocity vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Velocity (m/s)')

plt.legend()

plt.grid()

# Control inputs

plt.subplot(3, 2, 4)

plt.plot(time, controls[:, 0], label='Motor 1')

plt.plot(time, controls[:, 1], label='Motor 2')

plt.plot(time, controls[:, 2], label='Motor 3')

plt.plot(time, controls[:, 3], label='Motor 4')

plt.title('Motor Voltages vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Voltage (V)')

plt.legend()

plt.grid()

# 3D trajectory

ax = plt.subplot(3, 2, 5, projection='3d')

ax.plot(states[:, 0], states[:, 1], states[:, 2])

ax.set_title('3D Trajectory')

ax.set_xlabel('X (m)')

ax.set_ylabel('Y (m)')

ax.set_zlabel('Z (m)')

# Phase markers

for t in [5, 10, 15, 20, 25, 30, 35]:

idx = int(t / self.dt)

ax.scatter(states[idx, 0], states[idx, 1], states[idx, 2],

color='red', s=50, label=f'{t}s' if t == 5 else '')

# Error plot

plt.subplot(3, 2, 6)

target_x = np.piecewise(time,

[time < 15,

(15 <= time) & (time < 20),

(20 <= time) & (time < 25),

(25 <= time) & (time < 30),

(30 <= time) & (time < 35),

time >= 35],

[0, lambda t: 2*(t-15)/5, 2, lambda t: 2+2*(t-25)/5,

lambda t: 4-2*(t-30)/5, 2])

plt.plot(time, target_x - states[:, 0], label='X Error')

plt.plot(time, -states[:, 1], label='Y Error')

target_z = np.piecewise(time,

[time < 5,

(5 <= time) & (time < 35),

time >= 35],

[lambda t: 2*t/5, 2, lambda t: 2*(40-t)/5])

plt.plot(time, target_z - states[:, 2], label='Z Error')

plt.title('Position Errors')

plt.xlabel('Time (s)')

plt.ylabel('Error (m)')

plt.legend()

plt.grid()

plt.tight_layout()

plt.show()

# Run and plot simulation

simulator = DroneSimulator()

time, states, controls = simulator.simulate()

simulator.plot_results(time, states, controls)

Figure 7. position, attitude, motor voltage and velocity versus time

Figure 8. 3D trajectory and position errors

## 4.7 Use the code written in 4.6 repeating the movement described 4.5 and use on board camera to make the drawn land at a designated square of 0.1m by 0.1 10 Marks

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.patches import Rectangle

from scipy.integrate import odeint

class PIDController:

def __init__(self, kp, ki, kd, limit):

self.kp = kp

self.ki = ki

self.kd = kd

self.limit = limit

self.integral = 0

self.prev_error = 0

def update(self, error, dt):

self.integral += error * dt

derivative = (error - self.prev_error) / dt

output = self.kp * error + self.ki * self.integral + self.kd * derivative

self.prev_error = error

return np.clip(output, -self.limit, self.limit)

class DroneSimulator:

def __init__(self):

# Drone parameters

self.mass = 2.0       # kg

self.g = 9.81         # m/s²

self.kf = 0.08        # Thrust coefficient (N/V)

self.km = 0.01        # Torque coefficient (Nm/A)

self.arm_length = 0.2 # m

self.inertia = np.diag([0.01, 0.01, 0.02])  # kg·m²

# PID controllers

self.x_pid = PIDController(1.2, 0.05, 0.8, 2.0)

self.y_pid = PIDController(1.2, 0.05, 0.8, 2.0)

self.z_pid = PIDController(2.0, 0.1, 1.0, 3.0)

self.phi_pid = PIDController(0.8, 0.02, 0.4, 1.0)

self.theta_pid = PIDController(0.8, 0.02, 0.4, 1.0)

self.psi_pid = PIDController(0.5, 0.01, 0.3, 1.0)

# Initial state [x, y, z, vx, vy, vz, phi, theta, psi, p, q, r]

self.state = np.zeros(12)

self.time = 0

def dynamics(self, state, t, motor_voltages):

"""Drone dynamics model"""

x, y, z, vx, vy, vz, phi, theta, psi, p, q, r = state

V1, V2, V3, V4 = motor_voltages

# Calculate forces and torques

F1 = self.kf * V1

F2 = self.kf * V2

F3 = self.kf * V3

F4 = self.kf * V4

# Total thrust (in body frame)

thrust = F1 + F2 + F3 + F4

# Torques (roll, pitch, yaw)

tau_phi = self.arm_length * (F2 - F4)

tau_theta = self.arm_length * (F3 - F1)

tau_psi = self.km * (F1 - F2 + F3 - F4)

# Rotation matrix (body to world)

R = np.array([

[np.cos(theta)*np.cos(psi),

np.sin(phi)*np.sin(theta)*np.cos(psi)-np.cos(phi)*np.sin(psi),

np.cos(phi)*np.sin(theta)*np.cos(psi)+np.sin(phi)*np.sin(psi)],

[np.cos(theta)*np.sin(psi),

np.sin(phi)*np.sin(theta)*np.sin(psi)+np.cos(phi)*np.cos(psi),

np.cos(phi)*np.sin(theta)*np.sin(psi)-np.sin(phi)*np.cos(psi)],

[-np.sin(theta),

np.sin(phi)*np.cos(theta),

np.cos(phi)*np.cos(theta)]

])

# Translational dynamics

thrust_vector = np.array([0, 0, thrust])

gravity_vector = np.array([0, 0, -self.mass*self.g])

acceleration = (R @ thrust_vector + gravity_vector) / self.mass

# Rotational dynamics

omega = np.array([p, q, r])

tau = np.array([tau_phi, tau_theta, tau_psi])

omega_dot = np.linalg.inv(self.inertia) @ (tau - np.cross(omega, self.inertia @ omega))

# Kinematics

euler_rates = np.array([

p + q*np.sin(phi)*np.tan(theta) + r*np.cos(phi)*np.tan(theta),

q*np.cos(phi) - r*np.sin(phi),

(q*np.sin(phi) + r*np.cos(phi)) / np.cos(theta)

])

return np.concatenate((

[vx, vy, vz],

acceleration,

euler_rates,

omega_dot

))

def mission_control(self, t):

"""Mission sequence with PID control"""

if t < 5:

# Phase 1: Lift to 2m

target = [0, 0, min(2.0, 2.0*(t/5)), 0, 0, 0]

elif t < 10:

# Phase 2: Hover at 2m

target = [0, 0, 2.0, 0, 0, 0]

elif t < 15:

# Phase 3: Turn right (90 degrees)

target = [0, 0, 2.0, 0, 0, np.pi/2 * ((t-10)/5)]

elif t < 20:

# Phase 4: Move forward 2m

target = [2.0 * ((t-15)/5), 0, 2.0, 0, 0, np.pi/2]

elif t < 25:

# Phase 5: Turn left (90 degrees)

target = [2.0, 0, 2.0, 0, 0, np.pi/2 - np.pi/2 * ((t-20)/5)]

elif t < 30:

# Phase 6: Move forward 2m

target = [2.0 + 2.0 * ((t-25)/5), 0, 2.0, 0, 0, 0]

elif t < 35:

# Phase 7: Reverse 2m

target = [4.0 - 2.0 * ((t-30)/5), 0, 2.0, 0, 0, 0]

else:

# Phase 8: Soft landing

landing_time = max(0, 40 - t)

target = [2.0, 0, 2.0 * (landing_time/5), 0, 0, 0]

return self.pid_controller(target)

def pid_controller(self, target):

"""Full PID control for all axes"""

x_target, y_target, z_target, phi_target, theta_target, psi_target = target

x, y, z, _, _, _, phi, theta, psi, _, _, _ = self.state

# Position control

x_error = x_target - x

y_error = y_target - y

z_error = z_target - z

# Convert position errors to attitude targets

theta_target = self.x_pid.update(x_error, self.dt)

phi_target = -self.y_pid.update(y_error, self.dt)

thrust_adjust = self.z_pid.update(z_error, self.dt)

# Attitude control

phi_error = phi_target - phi

theta_error = theta_target - theta

psi_error = psi_target - psi

roll_adjust = self.phi_pid.update(phi_error, self.dt)

pitch_adjust = self.theta_pid.update(theta_error, self.dt)

yaw_adjust = self.psi_pid.update(psi_error, self.dt)

# Motor mixing

base_thrust = self.mass * self.g / (4 * self.kf)

V1 = base_thrust + thrust_adjust - roll_adjust + pitch_adjust + yaw_adjust

V2 = base_thrust + thrust_adjust + roll_adjust + pitch_adjust - yaw_adjust

V3 = base_thrust + thrust_adjust + roll_adjust - pitch_adjust + yaw_adjust

V4 = base_thrust + thrust_adjust - roll_adjust - pitch_adjust - yaw_adjust

return np.clip([V1, V2, V3, V4], 0, 12.0)

def simulate(self, dt=0.01, total_time=40):

"""Run the complete simulation"""

self.dt = dt

time_points = np.arange(0, total_time, dt)

states = []

controls = []

for t in time_points:

motor_voltages = self.mission_control(t)

state_dot = self.dynamics(self.state, t, motor_voltages)

self.state += state_dot * dt

states.append(self.state.copy())

controls.append(motor_voltages.copy())

return time_points, np.array(states), np.array(controls)

def plot_results(self, time, states, controls):

"""Plot simulation results"""

plt.figure(figsize=(18, 12))

# Position plot

plt.subplot(3, 2, 1)

plt.plot(time, states[:, 0], label='X')

plt.plot(time, states[:, 1], label='Y')

plt.plot(time, states[:, 2], label='Z')

plt.title('Position vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Position (m)')

plt.legend()

plt.grid()

# Attitude plot

plt.subplot(3, 2, 2)

plt.plot(time, np.degrees(states[:, 6]), label='Roll (φ)')

plt.plot(time, np.degrees(states[:, 7]), label='Pitch (θ)')

plt.plot(time, np.degrees(states[:, 8]), label='Yaw (ψ)')

plt.title('Attitude vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Angle (deg)')

plt.legend()

plt.grid()

# Velocity plot

plt.subplot(3, 2, 3)

plt.plot(time, states[:, 3], label='Vx')

plt.plot(time, states[:, 4], label='Vy')

plt.plot(time, states[:, 5], label='Vz')

plt.title('Velocity vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Velocity (m/s)')

plt.legend()

plt.grid()

# Control inputs

plt.subplot(3, 2, 4)

plt.plot(time, controls[:, 0], label='Motor 1')

plt.plot(time, controls[:, 1], label='Motor 2')

plt.plot(time, controls[:, 2], label='Motor 3')

plt.plot(time, controls[:, 3], label='Motor 4')

plt.title('Motor Voltages vs Time')

plt.xlabel('Time (s)')

plt.ylabel('Voltage (V)')

plt.legend()

plt.grid()

# 3D trajectory

ax = plt.subplot(3, 2, 5, projection='3d')

ax.plot(states[:, 0], states[:, 1], states[:, 2])

ax.set_title('3D Trajectory')

ax.set_xlabel('X (m)')

ax.set_ylabel('Y (m)')

ax.set_zlabel('Z (m)')

# Phase markers

for t in [5, 10, 15, 20, 25, 30, 35]:

idx = int(t / self.dt)

ax.scatter(states[idx, 0], states[idx, 1], states[idx, 2],

color='red', s=50, label=f'{t}s' if t == 5 else '')

# Error plot

plt.subplot(3, 2, 6)

target_x = np.piecewise(time,

[time < 15,

(15 <= time) & (time < 20),

(20 <= time) & (time < 25),

(25 <= time) & (time < 30),

(30 <= time) & (time < 35),

time >= 35],

[0, lambda t: 2*(t-15)/5, 2, lambda t: 2+2*(t-25)/5,

lambda t: 4-2*(t-30)/5, 2])

plt.plot(time, target_x - states[:, 0], label='X Error')

plt.plot(time, -states[:, 1], label='Y Error')

target_z = np.piecewise(time,

[time < 5,

(5 <= time) & (time < 35),

time >= 35],

[lambda t: 2*t/5, 2, lambda t: 2*(40-t)/5])

plt.plot(time, target_z - states[:, 2], label='Z Error')

plt.title('Position Errors')

plt.xlabel('Time (s)')

plt.ylabel('Error (m)')

plt.legend()

plt.grid()

plt.tight_layout()

plt.show()

class VisionLandingDrone(DroneSimulator):

def __init__(self):

super().__init__()

# Vision parameters

self.landing_target = np.array([2.0, 0.0])  # Center of landing zone (x,y)

self.landing_size = 0.1  # 0.1m x 0.1m target

self.camera_fov = np.radians(60)  # Camera field of view

self.detection_range = 3.0  # Max detection altitude (m)

# Landing controller

self.landing_pid = PIDController(1.5, 0.1, 0.5, 1.0)

def simulate_vision(self, position, altitude):

"""Simulate camera detection with noise"""

if altitude > self.detection_range:

return None  # Too high to detect

# Calculate position error with 1cm camera resolution

error = self.landing_target - position[:2]

detected_error = error + np.random.normal(0, 0.005, 2)  # Add 5mm noise

# Simulate limited FOV (can't see target if too far offset)

max_offset = altitude * np.tan(self.camera_fov/2)

if np.linalg.norm(detected_error) > max_offset:

return None

return detected_error

def mission_control(self, t):

"""Mission sequence with vision-assisted landing"""

if t < 5:

# Phase 1: Lift to 2m

target = [0, 0, min(2.0, 2.0*(t/5)), 0, 0, 0]

elif t < 10:

# Phase 2: Hover at 2m

target = [0, 0, 2.0, 0, 0, 0]

elif t < 15:

# Phase 3: Turn right (90 degrees)

target = [0, 0, 2.0, 0, 0, np.pi/2 * ((t-10)/5)]

elif t < 20:

# Phase 4: Move forward 2m

target = [2.0 * ((t-15)/5), 0, 2.0, 0, 0, np.pi/2]

elif t < 25:

# Phase 5: Turn left (90 degrees)

target = [2.0, 0, 2.0, 0, 0, np.pi/2 - np.pi/2 * ((t-20)/5)]

elif t < 30:

# Phase 6: Move forward 2m

target = [2.0 + 2.0 * ((t-25)/5), 0, 2.0, 0, 0, 0]

elif t < 35:

# Phase 7: Reverse 2m

target = [4.0 - 2.0 * ((t-30)/5), 0, 2.0, 0, 0, 0]

else:

# Phase 8: Vision-based landing

target = self.vision_landing_controller(t)

return self.pid_controller(target)

def vision_landing_controller(self, t):

"""Vision-assisted landing controller"""

current_z = self.state[2]

landing_time = max(0, 40 - t)

# Base target (slow descent)

target_z = 2.0 * (landing_time/5) if landing_time > 0 else 0

target_xy = self.landing_target.copy()

# Vision correction only when below detection range

if current_z <= self.detection_range:

vision_error = self.simulate_vision(self.state[:3], current_z)

if vision_error is not None:

# Apply correction with PID

xy_correction = self.landing_pid.update(vision_error, self.dt)

target_xy += xy_correction

return [target_xy[0], target_xy[1], target_z, 0, 0, 0]

def plot_landing_accuracy(self, states):

"""Plot final landing precision"""

landing_idx = int(40 / self.dt)

landing_pos = states[landing_idx:, :2]

target = self.landing_target

plt.figure(figsize=(8, 8))

plt.plot(landing_pos[:, 0], landing_pos[:, 1], 'b-', label='Drone Path')

plt.plot(target[0], target[1], 'rx', markersize=10, label='Target Center')

# Draw 0.1m x 0.1m landing square

landing_rect = Rectangle((target[0]-self.landing_size/2, target[1]-self.landing_size/2),

self.landing_size, self.landing_size,

linewidth=2, edgecolor='r', facecolor='none')

plt.gca().add_patch(landing_rect)

plt.title(f'Landing Precision (Final Error: {np.linalg.norm(landing_pos[-1] - target):.3f}m)')

plt.xlabel('X Position (m)')

plt.ylabel('Y Position (m)')

plt.axis('equal')

plt.grid()

plt.legend()

plt.show()

# Run simulation

drone = VisionLandingDrone()

time, states, controls = drone.simulate()

# Plot results

drone.plot_results(time, states, controls)

drone.plot_landing_accuracy(states)

# Print final landing error

final_pos = states[-1, :2]

target = drone.landing_target

error = np.linalg.norm(final_pos - target)

print(f"\nFinal Landing Error: {error*100:.1f} cm")

def plot_landing_accuracy(self, states):

"""Plot final landing precision with error checking"""

try:

# Find landing phase indices (last 20% of simulation)

landing_start = int(0.8 * len(states))

landing_pos = states[landing_start:, :2]

if len(landing_pos) == 0:

raise ValueError("No landing phase data available")

target = self.landing_target

final_error = np.linalg.norm(landing_pos[-1] - target)

plt.figure(figsize=(8, 8))

plt.plot(landing_pos[:, 0], landing_pos[:, 1], 'b-', label='Drone Path')

plt.plot(target[0], target[1], 'rx', markersize=10, label='Target Center')

# Draw landing square

landing_rect = Rectangle(

(target[0]-self.landing_size/2, target[1]-self.landing_size/2),

self.landing_size, self.landing_size,

linewidth=2, edgecolor='r', facecolor='none'

)

plt.gca().add_patch(landing_rect)

plt.title(f'Landing Precision (Final Error: {final_error*100:.1f} cm)')

plt.xlabel('X Position (m)')

plt.ylabel('Y Position (m)')

plt.axis('equal')

plt.grid()

plt.legend()

plt.show()

return final_error

except Exception as e:

print(f"Error plotting landing accuracy: {str(e)}")

return None

# Modified simulation call

try:

drone = VisionLandingDrone()

time, states, controls = drone.simulate(total_time=40)  # Ensure sufficient time

if len(states) > 0:

drone.plot_results(time, states, controls)

final_error = drone.plot_landing_accuracy(states)

if final_error is not None:

print(f"\nFinal Landing Error: {final_error*100:.1f} cm")

else:

print("Could not calculate landing accuracy")

else:

print("Simulation produced no data")

except Exception as e:

print(f"Simulation error: {str(e)}")

# Discussion

## 6.2.1 How do the theoretical simulations compare with practical results?

Theoretical Simulations

- Assumptions:

  - Idealized motor responses (no delays, perfect voltage-force conversion).

  - Simplified aerodynamics (ignores turbulence, ground effect).

  - Perfect sensor feedback (no noise, infinite resolution).

- Behaviour[2]:

  - Smooth, predictable trajectories.

  - PID control works optimally (no overshoot if tuned correctly).

  - Vision-based landing is precise (no detection errors).

Practical Results

- Challenges:

  - Motor/ESC delays: Real motors have response lags, affecting control stability.

  - Sensor noise: Gyroscopes/accelerometers introduce jitter, requiring filtering.

  - Environmental factors: Wind, uneven surfaces, and battery voltage drop impact performance.

  - Vision limitations: Camera latency, lighting changes, and motion blur affect landing precision.

- Observed Differences[3]:

  - Less smooth movements: Due to real-world disturbances.

  - Overshoot/oscillations: Tuned PID gains may need adjustment for hardware.

  - Landing inaccuracies: Vision systems may require multiple correction attempts.

Key Takeaways

  - Simulations provide a baseline for control design but must be refined with real-world testing.

  - Practical systems need robustness (e.g., sensor fusion, adaptive PID).

## 6.2.2 How does closing the loop with and adding the forward PID controller change the response of the flying robot car?

Open-Loop (No PID):

- Behaviour:

  - Unstable hover (small disturbances cause drift).

  - No correction for external forces (e.g., wind).

  - Inaccurate trajectory tracking (accumulated errors).

- Simulation Result[4]:

  - Drone fails to maintain altitude or reach waypoints precisely.

Closed-Loop PID[5]:

1. Proportional (P) Term:

  - Effect: Reduces steady-state error (e.g., maintains 2m altitude).

  - Trade-off: High `K_p` causes oscillations; low `K_p` leads to sluggish response.

2. Integral (I) Term:

  - Effect: Eliminates residual errors (e.g., compensates for battery drain over time).

  - Trade-off: High `K_i` causes integral windup (unstable corrections).

3. Derivative (D) Term:

  - Effect: Dampens oscillations (e.g., smoothens descent during landing).

  - Trade-off: High `K_d` amplifies sensor noise.

# References

[1]	M. Mariano and K. Scicluna, “Design and Implementation of an Electronic Speed Controller for Brushless DC motors,” 2017. doi: 10.13140/RG.2.2.17641.29286.

[2]	S. Bouabdallah and R. Siegwart, “Full control of a quadrotor,” in 2007 IEEE/RSJ International Conference on Intelligent Robots and Systems, Oct. 2007, pp. 153–158. doi: 10.1109/IROS.2007.4399042.

[3]	R. Mahony, V. Kumar, and P. Corke, “Multirotor Aerial Vehicles: Modeling, Estimation, and Control of Quadrotor,” IEEE Robotics & Automation Magazine, vol. 19, no. 3, pp. 20–32, Sep. 2012, doi: 10.1109/MRA.2012.2206474.

[4]	K. Alexis, G. Nikolakopoulos, and A. Tzes, “Model predictive quadrotor control: attitude, altitude and position experimental studies,” IET Control Theory & Applications, Aug. 2012, doi: 10.1049/iet-cta.2011.0348.

[5]	K. J. Åström and T. Hägglund, Advanced PID Control. ISA - The Instrumentation, Systems and Automation Society, 2006.