SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONICS ENGINEERING (BSEEE)

BEE 3206: CONTROL SYSTEMS_1 Lab 4

GROUP 5

Report By:

Mwongela Edward Aribu

142092

LECTURER: PROF. MWANGI MBUTHIA

LABORATORY TECHNOLOGIST: MR. JAMLICK NTOJIRA

21st December 2023

Table of Contents

List of figures	2

Title: Robot Car Racing Competition	3

Abstract	3

Introduction	3

KS0559(KS0559F) Keyestudio 4WD BT Multi-purpose Car V2.0	3

Electric motor rating	3

(Weight) Mass	3

Aerodynamic Drag	4

Methods and Materials	5

Assumptions	5

Objective	5

Procedure	5

Electric Racing Car code	7

MATLAB code	14

Results	17

Discussion and Conclusion	28

# List of figures

Figure 1	5

Figure 2	6

Figure 3	16

Figure 4	16

Figure 5 table showing the autocross speed and time values every 0.2 seconds for one lap.	26

Figure 6 lab 4 Matlab experimental graph	27

Figure 7 lab 3 Matlab experimental graph	27

# Title: Robot Car Racing Competition

# Abstract

The aim of this lab is to autonomously drive a robot car round a 40m racing track, determine the Robot car lap time, learn how to use a program with both speed and direction feedback to autonomously drive a robot car round 40m racing track.

# Introduction

## KS0559(KS0559F) Keyestudio 4WD BT Multi-purpose Car V2.0

The KS0559(KS0559F) Keyestudio 4WD BT Multi-purpose Car V2.0 is one of the most popular Arduino sets. It has a stunning appearance as well as powerful functions.  Furthermore, you can build your own projects or add other sensors to finish your research. Most of the components are connected by screws and copper pillars, allowing you to build your own robot in a few simple steps. Furthermore, we may master the fundamentals of line tracking, obstacle avoidance sensors, ultrasonic ranging, Bluetooth connection, motor drivers, and IR remote controls. This bundle includes 17 learning projects ranging from simple to sophisticated that will walk you through the process of building an Arduino robot.

## Electric motor rating

Power Output: A car's acceleration and top speed are directly influenced by the electric motor rating, which is commonly expressed in kW or HP. A more powerful motor will generate more torque and acceleration, allowing the vehicle to accelerate faster.

Torque: Electric motors instantly produce torque, which helps them accelerate quickly. More torque can be produced by motors with higher ratings, allowing for speedier acceleration from stopping position.

Efficiency: The electric motor's capacity to maintain high speeds can be impacted by its efficiency. More effective motors can operate at high speeds for extended periods of time while wasting less energy as heat.

## (Weight) Mass

Inertia: A car's mass influences its ability to accelerate and decelerate. Heavier vehicles have greater inertia, requiring more force to accelerate and stop. As a result, acceleration will be slower and braking distances will be greater.

Momentum: Once an automobile is moving, its mass has an impact on its capacity to maintain speed. Heavier vehicles have more momentum, making it more difficult to change speed or direction. This can influence a vehicle's agility and reactivity at greater speeds.

## Aerodynamic Drag

Aerodynamic drag is a force that resists the motion of the car as it passes through the air. The drag coefficient measures how aerodynamic the shape of a car is. Lower drag coefficients minimise the resistance the car faces, allowing it to travel faster while using less power.

Cross-Sectional Area: The frontal area of the car influences aerodynamic drag as well. Larger cross-sectional areas generate more air resistance, which limits top speed and may reduce acceleration.

Speed Restrictions: As an automobile speeds, aerodynamic drag increases. Overcoming air resistance at high speeds necessitates additional power. Cars with lower drag coefficients operate better at high speeds

# Methods and Materials

## Assumptions

- An Electric Racing Car is driven around a flat racing track shown in figure 1.

Figure 1

- The racing car is modelled as a simple Cart on wheels.

- Assume the Racing Car Longitudinal dynamics is modelled using the LDE where M is the lamped car mass, v is the car speed in m/s, b is the aerodynamic drag coefficient proportional to the speed and F is the force produced by the electric motor.

## Objective

- Autonomously drive a robot car round a 40m racing track

- Determine the Robot car lap time

- Learn how to use a program with both speed and direction feedback to autonomously drive a robot car round 40m racing track.

## Procedure

- You will use tape to mark out a 40m track on a surface of your choice (for the competition, all teams will use the Basketball Court). Use tape to mark the points A, B, C, D, L, E, F, G, H, and I as shown in Fig. 1

- Write code to autonomously drive the robot car on the track prepared in 4.2.1 along path A, B, C, D, L

- Write code to autonomously drive the robot car on the track prepared in 4.2.1 along the path A, B, C, D, E, F, G, H, I, A.

- Train your group robot car autonomous driving code on the racing track and make code adjustments to keep it within the track and achieve the best Lap time. Save values of speed and time every 0.2 seconds.

- Each group MUST upload a video of the best Skid Test and Autocross autonomous driving of your Robot Car and state the best lap time before the day of competition. Compare the lap time with the values obtained using MATLAB in Lab 3.

- Upload a table showing the autocross speed and time values every 0.2 seconds for one lap.

Figure 2

- Use the values recorded in 5.2.3 to plot the speed vs time values for the section A to B and draw the best curve fit during acceleration to steady state speed. Using your graph, obtain a new set of values of damping coefficient b (Ns/m) and motor Force F (N) for your robot car and compare them with the b and F values obtained in Lab_3.

- Include your autonomous driving code for your robot in the report.

# Electric Racing Car code

#define ML_Ctrl 4     // define the direction control pin of B motor

#define ML_PWM 5   //define the PWM control pin of B motor

#define MR_Ctrl 2    //define direction control pin of A motor

#define MR_PWM 6   //define the PWM control pin of A motor

#define encoderPin 9 //define encoder input pin

int pulsesPerRevolution = 20; // Number of pulses per revolution of the wheel

float wheelCircumference = 20.4; // Wheel circumference in cm

float arcLength = 1.02; // Arc length in cm [circumference/(holes)]

float totalDistance = 0; //Total Distance in m

unsigned long StartMillis = 0;

unsigned long stopMillis = 0;

unsigned long period = 0;

volatile bool B_State = 0;

volatile bool A_State = 0;

volatile byte B_PWM = 0;

volatile byte A_PWM = 0;

volatile bool Start = 0;

bool S2 = 0;

bool S3 = 0;

bool S4 = 0;

bool S5 = 0;

bool S6 = 0;

bool S7 = 0;

bool S8 = 0;

bool S9 = 0;

float S1_Distance = 5;

float S2_Distance = 1.1;

float S3_Distance = 8;

float S4_Distance = 1.1;

float S5_Distance = 10.0;

float S6_Distance = 1.;

float S7_Distance = 8.0;

float S8_Distance = 1.1;

float S9_Distance = 5.0;

bool stop = 1;

void setup()

{

Serial.begin(9600);

pinMode(ML_Ctrl, OUTPUT);//set direction control pin of B motor to output

pinMode(ML_PWM, OUTPUT);//set PWM control pin of B motor to output

pinMode(MR_Ctrl, OUTPUT);//set direction control pin of A motor to output

pinMode(MR_PWM, OUTPUT);//set the PWM control pin of A motor to output

pinMode(encoderPin, INPUT);

digitalWrite(ML_Ctrl, HIGH);

digitalWrite(MR_Ctrl, HIGH);

}

void loop()

{

StartMillis = millis();

if (stop && !Start) //5

{

totalDistance = 0;

//Start

//Serial.println("Start: Sector 1");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 100);

digitalWrite(MR_Ctrl, HIGH);

analogWrite(MR_PWM, 104);

Start = 1;

}

else if (Start && totalDistance >= S1_Distance && !S2 ) //2.2

{

//Turn

//Serial.println("Turn 180: Sector 2");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 120); //B

digitalWrite(MR_Ctrl, HIGH);

analogWrite(MR_PWM, 73); //

S2 = 1;

}

else if (Start && S2 && totalDistance >= (S1_Distance + S2_Distance) && !S3) //8

{

// Sector 3

//Serial.println("Sector 3");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 100);

digitalWrite(MR_Ctrl,HIGH);

analogWrite(MR_PWM, 104);

S3 = 1;

}

else if (Start && S2 && S3 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance) && !S4) //2.2

{

//Turn

//Serial.println("Turn 180: Sector 2");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 120); //B

digitalWrite(MR_Ctrl, HIGH);

analogWrite(MR_PWM, 73); //

S4 = 1;

}

else if (Start && S2 && S3 && S4 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance) && !S5) //10.0

{

// Sector 3

//Serial.println("Sector 3");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 100);

digitalWrite(MR_Ctrl,HIGH);

analogWrite(MR_PWM, 104);

S5 = 1;

}

else if (Start && S2 && S3 && S4 && S5 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance + S5_Distance) && !S6) //2.2

{

//Turn

//Serial.println("Turn 180: Sector 2");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 127); //B

digitalWrite(MR_Ctrl, HIGH);

analogWrite(MR_PWM, 73); //

S6 = 1;

}

else if (Start && S2 && S3 && S4 && S5 && S6 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance + S5_Distance + S6_Distance) && !S7) //8.0

{

// Sector 3

//Serial.println("Sector 3");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 100);

digitalWrite(MR_Ctrl,HIGH);

analogWrite(MR_PWM, 104);

S7 = 1;

}

else if (Start && S2 && S3 && S4 && S5 && S6 && S7 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance + S5_Distance + S6_Distance + S7_Distance) && !S8) //2.2

{

//Turn

//Serial.println("Turn 180: Sector 2");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 120); //B

digitalWrite(MR_Ctrl, HIGH);

analogWrite(MR_PWM, 73); //

S8 = 1;

}

else if (Start && S2 && S3 && S4 && S5 && S6 && S7 && S8 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance + S5_Distance + S6_Distance + S7_Distance + S8_Distance) && !S9) //5

{

// Sector 3

//Serial.println("Sector 3");

digitalWrite(ML_Ctrl, HIGH);

analogWrite(ML_PWM, 100);

digitalWrite(MR_Ctrl,HIGH);

analogWrite(MR_PWM, 104);

S9 = 1;

}

else if (Start && S2 && S3 && S4 && S5 && S6 && S7 && S8 && S9 && totalDistance >= (S1_Distance + S2_Distance + S3_Distance + S4_Distance + S5_Distance + S6_Distance + S7_Distance + S8_Distance + S9_Distance))

{

//Stop

//Serial.println("Stop");

B_State = 0;

A_State = 0;

B_PWM = 0;

A_PWM = 0;

digitalWrite(ML_Ctrl, LOW);

analogWrite(ML_PWM, 0);

digitalWrite(MR_Ctrl, LOW);

analogWrite(MR_PWM, 0);

stop = 0;

Start = 0;

}

// Measure the time duration of a HIGH pulse on the encoder pin

unsigned long durationHigh = pulseIn(encoderPin, HIGH, 2000000);

unsigned long durationLow = pulseIn(encoderPin, LOW, 2000000);

unsigned long duration = durationLow + durationHigh;

// Calculate speed (metres per second)

float speed = ((arcLength * 10000) / duration);

// Print results

//Serial.print("Speed (m/s): ");

Serial.print(speed, 4);

// Calculate distance (cm) based on the number of pulses per revolution

stopMillis = millis();

period = stopMillis - StartMillis;

if (duration >= (period * 500))

{

float distance = ((period * arcLength) / duration) * 10.0;

//Serial.print("\tDistance (m): ");

Serial.print("\t");

Serial.print(distance, 4);

totalDistance = totalDistance + distance;

//Serial.print("\tTotal distance (m): ");

Serial.print("\t");

Serial.println(totalDistance, 4);

}

else

{

float distance = ((period * arcLength) / (period * 500)) * 10.0;

//Serial.print("\tDistance (m): ");

Serial.print("\t");

Serial.print(distance, 4);

totalDistance = totalDistance + distance;

//Serial.print("\tTotal distance (m): ");

Serial.print("\t");

Serial.println(totalDistance, 4);

}

}

# MATLAB code

>> % Define the model equation with a fixed value for m (0.636 in this case)

model = @(params, t) params(1)./params(2) * (1 - exp(-params(2)*t/0.636));

% Provided experimental data

t = [0.0, 0.616, 0.819, 1.096, 1.219, 1.421, 1.632, 1.828, 1.991, 2.236, 2.441, 2.606, 2.837, 3.01, 3.271, 3.403, 3.600, 3.811, 4.068, 4.272, 4.405, 4.608, 4.801, 5.078, 5.209, 5.484, 5.685, 5.809, 6.09, 6.212];

v = [0.2758, 0.2758, 0.3225, 0.3225, 0.3225, 0.3339, 0.3339, 0.3346, 0.3262, 0.2928, 0.303, 0.3278, 0.2898, 0.2898, 0.303, 0.2912, 0.303, 0.3329, 0.3329, 0.3322, 0.3179, 0.2942, 0.2975, 0.2975, 0.3383, 0.2758, 0.2758, 0.3225, 0.3225, 0.3225];

% Use lsqcurvefit to fit the model to the data with initial guesses

initial_guess = [1.0, 1.0]; % You might need to adjust these initial guesses

params = lsqcurvefit(@(params, t) model(params, t), initial_guess, t, v);

% Extract the fitted parameters

F_fit = params(1);

b_fit = params(2);

% Generate a higher-resolution time vector

t_highres = linspace(min(t), max(t), 1000);

% Generate theoretical values using the fitted parameters

v_fit = model(params, t_highres);

% Plot the experimental data and the fitted curve

figure;

scatter(t, v, 'o', 'DisplayName', 'Experimental Data');

hold on;

plot(t_highres, v_fit, 'LineWidth', 2, 'DisplayName', 'Fitted Curve');

xlabel('Time (s)');

ylabel('Velocity (m/s)');

title('Wheel On Carts');

% Add annotations for F_fit and b_fit

annotation('textbox', [0.6, 0.4, 0.25, 0.1], 'String', {['F_{fit} = ', num2str(F_fit), ' N'], ['b_{fit} = ', num2str(b_fit),' Ns/m']}, 'EdgeColor', 'none');

% Adjust the legend position

legend('show', 'Location', 'Best');

grid on;

% Use the fitted parameters to calculate velocity at each point in time

v_lap = model(params, t);

% Integrate velocity to obtain displacement

d_lap = cumtrapz(t, v_lap);

% Find the time when displacement is close to zero (indicating one lap completion)

tolerance = 0.001; % Adjust based on the specifics of your track

lap_time = t(find(abs(d_lap) < tolerance, 1));

% Display Lap Time

disp(['Lap Time: ', num2str(lap_time), ' seconds']);

Figure 3

Figure 4

# Results

| minutes | seconds | speed | Timestamp |
| --- | --- | --- | --- |
| 55 | 20.494 | 0 | 14:55:20.494 |
| 55 | 20.616 | 0.2758 | 14:55:20.616 |
| 55 | 20.819 | 0.3225 | 14:55:20.819 |
| 55 | 21.096 | 0.3225 | 14:55:21.096 |
| 55 | 21.219 | 0.3225 | 14:55:21.219 |
| 55 | 21.421 | 0.3339 | 14:55:21.421 |
| 55 | 21.632 | 0.3339 | 14:55:21.632 |
| 55 | 21.828 | 0.3346 | 14:55:21.828 |
| 55 | 21.991 | 0.3262 | 14:55:21.991 |
| 55 | 22.236 | 0.2928 | 14:55:22.236 |
| 55 | 22.441 | 0.303 | 14:55:22.441 |
| 55 | 22.606 | 0.3278 | 14:55:22.606 |
| 55 | 22.837 | 0.2898 | 14:55:22.837 |
| 55 | 23.010 | 0.2898 | 14:55:23.010 |
| 55 | 23.271 | 0.303 | 14:55:23.271 |
| 55 | 23.403 | 0.2912 | 14:55:23.403 |
| 55 | 23.600 | 0.303 | 14:55:23.600 |
| 55 | 23.811 | 0.3329 | 14:55:23.811 |
| 55 | 24.068 | 0.3329 | 14:55:24.068 |
| 55 | 24.272 | 0.3322 | 14:55:24.272 |
| 55 | 24.405 | 0.3179 | 14:55:24.405 |
| 55 | 24.608 | 0.2942 | 14:55:24.608 |
| 55 | 24.801 | 0.2975 | 14:55:24.801 |
| 55 | 25.078 | 0.2975 | 14:55:25.078 |
| 55 | 25.209 | 0.3383 | 14:55:25.209 |
| 55 | 25.484 | 0.2758 | 14:55:25.484 |
| 55 | 25.685 | 0.2758 | 14:55:25.685 |
| 55 | 25.809 | 0.3225 | 14:55:25.809 |
| 55 | 26.090 | 0.3225 | 14:55:26.090 |
| 55 | 26.212 | 0.3225 | 14:55:26.212 |
| 55 | 26.416 | 0.3225 | 14:55:26.416 |
| 55 | 26.620 | 0.3339 | 14:55:26.620 |
| 55 | 26.875 | 0.2917 | 14:55:26.875 |
| 55 | 27.078 | 0.3346 | 14:55:27.078 |
| 55 | 27.281 | 0.2969 | 14:55:27.281 |
| 55 | 27.446 | 0.2928 | 14:55:27.446 |
| 55 | 27.612 | 0.303 | 14:55:27.612 |
| 55 | 27.847 | 0.3278 | 14:55:27.847 |
| 55 | 28.011 | 0.3345 | 14:55:28.011 |
| 55 | 28.215 | 0.2898 | 14:55:28.215 |
| 55 | 28.460 | 0.2772 | 14:55:28.460 |
| 55 | 28.654 | 0.3084 | 14:55:28.654 |
| 55 | 28.897 | 0.303 | 14:55:28.897 |
| 55 | 29.059 | 0.303 | 14:55:29.059 |
| 55 | 29.261 | 0.3329 | 14:55:29.261 |
| 55 | 29.465 | 0.3322 | 14:55:29.465 |
| 55 | 29.630 | 0.3179 | 14:55:29.630 |
| 55 | 29.869 | 0.2975 | 14:55:29.869 |
| 55 | 30.066 | 0.2975 | 14:55:30.066 |
| 55 | 30.267 | 0.3383 | 14:55:30.267 |
| 55 | 30.451 | 0.2758 | 14:55:30.451 |
| 55 | 30.675 | 0.2758 | 14:55:30.675 |
| 55 | 30.807 | 0.3225 | 14:55:30.807 |
| 55 | 31.008 | 0.3225 | 14:55:31.008 |
| 55 | 31.279 | 0.3225 | 14:55:31.279 |
| 55 | 31.424 | 0.3225 | 14:55:31.424 |
| 55 | 31.606 | 0.3339 | 14:55:31.606 |
| 55 | 31.807 | 0.2917 | 14:55:31.807 |
| 55 | 32.003 | 0.3346 | 14:55:32.003 |
| 55 | 32.208 | 0.3262 | 14:55:32.208 |
| 55 | 32.411 | 0.2928 | 14:55:32.411 |
| 55 | 32.615 | 0.303 | 14:55:32.615 |
| 55 | 32.818 | 0.3394 | 14:55:32.818 |
| 55 | 33.023 | 0.2898 | 14:55:33.023 |
| 55 | 33.220 | 0.2772 | 14:55:33.220 |
| 55 | 33.422 | 0.3084 | 14:55:33.422 |
| 55 | 33.601 | 0.303 | 14:55:33.601 |
| 55 | 33.818 | 0.303 | 14:55:33.818 |
| 55 | 34.015 | 0.3329 | 14:55:34.015 |
| 55 | 34.246 | 0.3322 | 14:55:34.246 |
| 55 | 34.448 | 0.3179 | 14:55:34.448 |
| 55 | 34.692 | 0.2942 | 14:55:34.692 |
| 55 | 34.845 | 0.2975 | 14:55:34.845 |
| 55 | 35.046 | 0.2975 | 14:55:35.046 |
| 55 | 35.224 | 0.3383 | 14:55:35.224 |
| 55 | 35.446 | 0.2758 | 14:55:35.446 |
| 55 | 35.649 | 0.3225 | 14:55:35.649 |
| 55 | 35.809 | 0.3225 | 14:55:35.809 |
| 55 | 36.009 | 0.3225 | 14:55:36.009 |
| 55 | 36.233 | 0.3339 | 14:55:36.233 |
| 55 | 36.437 | 0.2917 | 14:55:36.437 |
| 55 | 36.658 | 0.2917 | 14:55:36.658 |
| 55 | 36.824 | 0.3346 | 14:55:36.824 |
| 55 | 37.060 | 0.2969 | 14:55:37.060 |
| 55 | 37.224 | 0.2928 | 14:55:37.224 |
| 55 | 37.469 | 0.303 | 14:55:37.469 |
| 55 | 37.673 | 0.3278 | 14:55:37.673 |
| 55 | 37.805 | 0.3394 | 14:55:37.805 |
| 55 | 38.081 | 0.2898 | 14:55:38.081 |
| 55 | 38.274 | 0.2772 | 14:55:38.274 |
| 55 | 38.416 | 0.303 | 14:55:38.416 |
| 55 | 38.681 | 0.2912 | 14:55:38.681 |
| 55 | 38.876 | 0.303 | 14:55:38.876 |
| 55 | 39.075 | 0.3329 | 14:55:39.075 |
| 55 | 39.247 | 0.3329 | 14:55:39.247 |
| 55 | 39.402 | 0.3322 | 14:55:39.402 |
| 55 | 39.666 | 0.3179 | 14:55:39.666 |
| 55 | 39.832 | 0.2942 | 14:55:39.832 |
| 55 | 40.025 | 0.2975 | 14:55:40.025 |
| 55 | 40.227 | 0.2758 | 14:55:40.227 |
| 55 | 40.430 | 0.2758 | 14:55:40.430 |
| 55 | 40.609 | 0.3225 | 14:55:40.609 |
| 55 | 40.828 | 0.3225 | 14:55:40.828 |
| 55 | 41.095 | 0.3225 | 14:55:41.095 |
| 55 | 41.246 | 0.3225 | 14:55:41.246 |
| 55 | 41.450 | 0.3339 | 14:55:41.450 |
| 55 | 41.652 | 0.2917 | 14:55:41.652 |
| 55 | 41.845 | 0.3346 | 14:55:41.845 |
| 55 | 42.049 | 0.2928 | 14:55:42.049 |
| 55 | 42.248 | 0.303 | 14:55:42.248 |
| 55 | 42.453 | 0.3407 | 14:55:42.453 |
| 55 | 42.657 | 0.3394 | 14:55:42.657 |
| 55 | 42.846 | 0.2898 | 14:55:42.846 |
| 55 | 43.045 | 0.2772 | 14:55:43.045 |
| 55 | 43.249 | 0.303 | 14:55:43.249 |
| 55 | 43.453 | 0.303 | 14:55:43.453 |
| 55 | 43.640 | 0.3329 | 14:55:43.640 |
| 55 | 43.881 | 0.3329 | 14:55:43.881 |
| 55 | 44.054 | 0.3179 | 14:55:44.054 |
| 55 | 44.205 | 0.3179 | 14:55:44.205 |
| 55 | 44.407 | 0.2942 | 14:55:44.407 |
| 55 | 44.611 | 0.2975 | 14:55:44.611 |
| 55 | 44.802 | 0.2758 | 14:55:44.802 |
| 55 | 45.049 | 0.3225 | 14:55:45.049 |
| 55 | 45.205 | 0.3225 | 14:55:45.205 |
| 55 | 45.407 | 0.3339 | 14:55:45.407 |
| 55 | 45.610 | 0.2917 | 14:55:45.610 |
| 55 | 45.805 | 0.3262 | 14:55:45.805 |
| 55 | 46.036 | 0.2928 | 14:55:46.036 |
| 55 | 46.239 | 0.3278 | 14:55:46.239 |
| 55 | 46.401 | 0.3345 | 14:55:46.401 |
| 55 | 46.603 | 0.2772 | 14:55:46.603 |
| 55 | 46.807 | 0.2912 | 14:55:46.807 |
| 55 | 47.009 | 0.3329 | 14:55:47.009 |
| 55 | 47.212 | 0.3179 | 14:55:47.212 |
| 55 | 47.408 | 0.2975 | 14:55:47.408 |
| 55 | 47.613 | 0.2758 | 14:55:47.613 |
| 55 | 47.859 | 0.3225 | 14:55:47.859 |
| 55 | 48.022 | 0.3225 | 14:55:48.022 |
| 55 | 48.226 | 0.3339 | 14:55:48.226 |
| 55 | 48.431 | 0.3346 | 14:55:48.431 |
| 55 | 48.635 | 0.2928 | 14:55:48.635 |
| 55 | 48.825 | 0.3407 | 14:55:48.825 |
| 55 | 49.089 | 0.3345 | 14:55:49.089 |
| 55 | 49.229 | 0.2898 | 14:55:49.229 |
| 55 | 49.404 | 0.2772 | 14:55:49.404 |
| 55 | 49.617 | 0.303 | 14:55:49.617 |
| 55 | 49.839 | 0.2912 | 14:55:49.839 |
| 55 | 50.007 | 0.303 | 14:55:50.007 |
| 55 | 50.273 | 0.3329 | 14:55:50.273 |
| 55 | 50.487 | 0.2907 | 14:55:50.487 |
| 55 | 50.667 | 0.3179 | 14:55:50.667 |
| 55 | 50.867 | 0.2942 | 14:55:50.867 |
| 55 | 51.066 | 0.2975 | 14:55:51.066 |
| 55 | 51.229 | 0.2975 | 14:55:51.229 |
| 55 | 51.422 | 0.2758 | 14:55:51.422 |
| 55 | 51.627 | 0.2758 | 14:55:51.627 |
| 55 | 51.867 | 0.3225 | 14:55:51.867 |
| 55 | 52.016 | 0.3225 | 14:55:52.016 |
| 55 | 52.212 | 0.3225 | 14:55:52.212 |
| 55 | 52.419 | 0.3339 | 14:55:52.419 |
| 55 | 52.606 | 0.2917 | 14:55:52.606 |
| 55 | 52.824 | 0.2917 | 14:55:52.824 |
| 55 | 53.003 | 0.3346 | 14:55:53.003 |
| 55 | 53.225 | 0.3262 | 14:55:53.225 |
| 55 | 53.470 | 0.2928 | 14:55:53.470 |
| 55 | 53.635 | 0.303 | 14:55:53.635 |
| 55 | 53.837 | 0.3407 | 14:55:53.837 |
| 55 | 54.029 | 0.3394 | 14:55:54.029 |
| 55 | 54.206 | 0.2898 | 14:55:54.206 |
| 55 | 54.417 | 0.2772 | 14:55:54.417 |
| 55 | 54.618 | 0.3084 | 14:55:54.618 |
| 55 | 54.822 | 0.303 | 14:55:54.822 |
| 55 | 55.022 | 0.303 | 14:55:55.022 |
| 55 | 55.290 | 0.3329 | 14:55:55.290 |
| 55 | 55.443 | 0.2907 | 14:55:55.443 |
| 55 | 55.646 | 0.3179 | 14:55:55.646 |
| 55 | 55.851 | 0.2758 | 14:55:55.851 |
| 55 | 56.088 | 0.3225 | 14:55:56.088 |
| 55 | 56.287 | 0.3225 | 14:55:56.287 |
| 55 | 56.454 | 0.3225 | 14:55:56.454 |
| 55 | 56.654 | 0.3225 | 14:55:56.654 |
| 55 | 56.859 | 0.3225 | 14:55:56.859 |
| 55 | 56.074 | 0.3339 | 14:55:56.074 |
| 55 | 57.269 | 0.2917 | 14:55:57.269 |
| 55 | 57.480 | 0.3346 | 14:55:57.480 |
| 55 | 57.664 | 0.3262 | 14:55:57.664 |
| 55 | 57.869 | 0.2928 | 14:55:57.869 |
| 55 | 58.015 | 0.303 | 14:55:58.015 |
| 55 | 58.220 | 0.3407 | 14:55:58.220 |
| 55 | 58.401 | 0.3278 | 14:55:58.401 |
| 55 | 58.645 | 0.2898 | 14:55:58.645 |
| 55 | 58.808 | 0.2772 | 14:55:58.808 |
| 55 | 59.007 | 0.303 | 14:55:59.007 |
| 55 | 59.205 | 0.2912 | 14:55:59.205 |
| 55 | 59.405 | 0.303 | 14:55:59.405 |
| 56 | 59.609 | 0.3329 | 14:55:59.609 |
| 56 | 59.811 | 0.2907 | 14:55:59.811 |
| 56 | 00.016 | 0.3179 | 14:56:00.016 |
| 56 | 00.213 | 0.2942 | 14:56:00.213 |
| 56 | 00.403 | 0.2975 | 14:56:00.403 |
| 56 | 00.682 | 0.3179 | 14:56:00.682 |
| 56 | 00.886 | 0.2942 | 14:56:00.886 |
| 56 | 01.039 | 0.2975 | 14:56:01.039 |
| 56 | 01.222 | 0.2975 | 14:56:01.222 |
| 56 | 01.497 | 0.2758 | 14:56:01.497 |
| 56 | 01.619 | 0.3225 | 14:56:01.619 |
| 56 | 01.883 | 0.3225 | 14:56:01.883 |
| 56 | 02.028 | 0.3225 | 14:56:02.028 |
| 56 | 02.214 | 0.3225 | 14:56:02.214 |
| 56 | 02.435 | 0.3225 | 14:56:02.435 |
| 56 | 02.638 | 0.3339 | 14:56:02.638 |
| 56 | 02.843 | 0.2917 | 14:56:02.843 |
| 56 | 03.088 | 0.3346 | 14:56:03.088 |
| 56 | 03.250 | 0.3262 | 14:56:03.250 |
| 56 | 03.455 | 0.2928 | 14:56:03.455 |
| 56 | 03.661 | 0.303 | 14:56:03.661 |
| 56 | 03.862 | 0.3407 | 14:56:03.862 |
| 56 | 04.005 | 0.3345 | 14:56:04.005 |
| 56 | 04.229 | 0.2898 | 14:56:04.229 |
| 56 | 04.456 | 0.2772 | 14:56:04.456 |
| 56 | 04.641 | 0.303 | 14:56:04.641 |
| 56 | 04.862 | 0.3084 | 14:56:04.862 |
| 56 | 05.005 | 0.303 | 14:56:05.105 |
| 56 | 05.268 | 0.3329 | 14:56:05.268 |
| 56 | 05.465 | 0.2907 | 14:56:05.465 |
| 56 | 05.609 | 0.3179 | 14:56:05.609 |
| 56 | 05.869 | 0.2942 | 14:56:05.869 |
| 56 | 06.047 | 0.2975 | 14:56:06.047 |
| 56 | 06.268 | 0.2975 | 14:56:06.268 |
| 56 | 06.416 | 0.2758 | 14:56:06.416 |
| 56 | 06.681 | 0.2758 | 14:56:06.681 |
| 56 | 06.804 | 0.3225 | 14:56:06.804 |
| 56 | 07.088 | 0.3225 | 14:56:07.088 |
| 56 | 07.210 | 0.3225 | 14:56:07.210 |
| 56 | 07.404 | 0.3225 | 14:56:07.404 |
| 56 | 07.690 | 0.3339 | 14:56:07.690 |
| 56 | 07.813 | 0.2917 | 14:56:07.813 |
| 56 | 08.017 | 0.3346 | 14:56:08.017 |
| 56 | 08.221 | 0.3262 | 14:56:08.221 |
| 56 | 08.406 | 0.2928 | 14:56:08.406 |
| 56 | 08.629 | 0.303 | 14:56:08.629 |
| 56 | 08.811 | 0.3278 | 14:56:08.811 |
| 56 | 09.045 | 0.3345 | 14:56:09.045 |
| 56 | 09.228 | 0.2898 | 14:56:09.228 |
| 56 | 09.430 | 0.2772 | 14:56:09.430 |
| 56 | 09.635 | 0.303 | 14:56:09.635 |
| 56 | 09.839 | 0.2912 | 14:56:09.839 |
| 56 | 10.039 | 0.303 | 14:56:10.039 |
| 56 | 10.204 | 0.3329 | 14:56:10.204 |
| 56 | 10.426 | 0.3322 | 14:56:10.426 |
| 56 | 10.610 | 0.2758 | 14:56:10.610 |
| 56 | 10.831 | 0.2758 | 14:56:10.831 |
| 56 | 11.033 | 0.3225 | 14:56:11.033 |
| 56 | 11.220 | 0.3225 | 14:56:11.220 |
| 56 | 11.435 | 0.3225 | 14:56:11.435 |
| 56 | 11.639 | 0.3225 | 14:56:11.639 |
| 56 | 11.841 | 0.3225 | 14:56:11.841 |
| 56 | 12.005 | 0.3339 | 14:56:12.005 |
| 56 | 12.241 | 0.2917 | 14:56:12.241 |
| 56 | 12.425 | 0.3346 | 14:56:12.425 |
| 56 | 12.648 | 0.3262 | 14:56:12.648 |
| 56 | 12.812 | 0.2928 | 14:56:12.812 |
| 56 | 13.057 | 0.303 | 14:56:13.057 |
| 56 | 13.261 | 0.3278 | 14:56:13.261 |
| 56 | 13.415 | 0.3394 | 14:56:13.415 |
| 56 | 13.657 | 0.2898 | 14:56:13.657 |
| 56 | 13.819 | 0.2772 | 14:56:13.819 |
| 56 | 14.059 | 0.303 | 14:56:14.059 |
| 56 | 14.215 | 0.2912 | 14:56:14.215 |
| 56 | 14.417 | 0.303 | 14:56:14.417 |
| 56 | 17.174 | 0 | 14:56:17.174 |

Figure 5 table showing the autocross speed and time values every 0.2 seconds for one lap.

Figure 6 lab 4 Matlab experimental graph

Figure 7 lab 3 Matlab experimental graph

# Discussion and Conclusion

The model is described by the equation:

After applying the least squares curve fitting algorithm to the experimental data, we obtained the following fitted parameters:

These parameters represent the force and damping coefficient, respectively, in the context of the model. The curve generated by the fitted model closely matches the experimental data, as illustrated in the plotted graph.

This analysis provides insights into the dynamics of the system under investigation, particularly the forces involved and the damping effect on the velocity-time relationship. The accuracy of the fitted parameters depends on the quality of the experimental data and the appropriateness of the chosen model. Further experiments and analysis may refine these findings and contribute to a more comprehensive understanding of the system.

Top of Form