SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONIC ENGINEERING

MULTIVARIATE CONTROL - LAB 3 REPORT

Simulation and Practical Implementation of CKF + PI Control of a Quadcopter (X Configuration)

By

Edward Aribu – 142092

Collaborators:

Alvin Nyandeje, Andrew Nzioki, Leo Mulama, Kimutai Ronny Kibiwott

Lecturer: Prof Mbuthia

Supervisor: Mr. Jamlick Ntonjira

Date: January 2026

Contents

List of figures	3

Abstract	4

1. Introduction	5

2. Preparations	5

2.1 Hardware Configuration and Assembly	5

2.2 System Parameterisation	6

3. Mathematical Modelling of the System	Error! Bookmark not defined.

3.1 Electromechanical Actuator Dynamics	Error! Bookmark not defined.

3.2 Control Allocation and X-Configuration Torques	Error! Bookmark not defined.

3.3 Rigid Body Kinematics and Dynamics	Error! Bookmark not defined.

4. Laboratory Procedures and Observations	Error! Bookmark not defined.

4.1 Phase 1: Altitude-Only Hover Simulation (CKF + PI)	Error! Bookmark not defined.

4.2 Phase 2: Cascaded Altitude + Attitude Simulation (CKF + PI)	Error! Bookmark not defined.

4.3 Phase 3: Practical Implementation on ESP32-CAM	Error! Bookmark not defined.

5. Individual Laboratory Analysis	Error! Bookmark not defined.

5.1 Why CKF is Preferred over EKF for this System	Error! Bookmark not defined.

5.2 Impact of Ignoring Motor Dynamics	Error! Bookmark not defined.

5.3 How Euler Discretisation Affects Stability	Error! Bookmark not defined.

5.4 Effect of Increasing Sampling Time	Error! Bookmark not defined.

5.5 Cascaded Control Topology and Bandwidth Separation	Error! Bookmark not defined.

5.6 Sources of Error and Their Effects	Error! Bookmark not defined.

6. Conclusion	Error! Bookmark not defined.

7. References	Error! Bookmark not defined.

## List of figures

Table 1: Quadcopter Rigid Body, Kinematic, and Environmental Parameters	6

Table 2: Brushless DC (BLDC) Motor and Propeller Electromechanical Parameters	7

Table 3: Summary of Achieved Performance Metrics	9

# Abstract

The rigorous modeling, simulation, and empirical hardware implementation of a highly coupled, twenty-four-state X-configuration quadcopter were successfully executed to analyse the efficacy of nonlinear state estimation and multivariate cascaded control loops. Utilizing an ESP32-CAM microcontroller and four independent brushless DC (BLDC) motors driven by electronic speed controllers (ESCs), the physical system was stabilized using a Proportional-Integral (PI) control architecture augmented by a Cubature Kalman Filter (CKF). The continuous-time rigid-body and electromechanical differential equations of motion were digitized utilizing a fixed-step Forward Euler numerical integration technique operating at 250 Hz. Comprehensive Python simulations validated the mathematical framework, achieving a transient height overshoot strictly below 10%, an altitude settling time under 3.0 seconds, a forward velocity tracking error below 5%, and a steady-state attitude error converging asymptotically to zero. The analysis confirmed that the CKF provides superior computational stability and estimation accuracy over the Extended Kalman Filter (EKF) by eliminating reliance on Jacobian-based linearization.

# 1. Introduction

The autonomous operation of unmanned aerial vehicles (UAVs), particularly vertical take-off and landing (VTOL) multirotor systems such as quadcopters, represents a paramount challenge in contemporary multivariate control theory. A standard quadrotor vehicle operates with six degrees of freedom (6-DOF)—comprising three translational spatial coordinates and three rotational Euler angles—yet it is inherently underactuated, possessing only four independent control inputs generated by the thrust of its fixed-pitch rotors. Consequently, the execution of complex manoeuvres requires deliberate induction of highly coupled, cross-axis aerodynamic forces, mandating a control architecture capable of resolving intense non-linearities, gyroscopic coupling, and susceptibility to exogenous environmental disturbances.

Historically, fundamental flight control methodologies relied heavily on simplified linear models derived by evaluating the vehicle strictly about its steady-state hover equilibrium point. While classical Proportional-Integral-Derivative (PID) and Linear-Quadratic Regulator (LQR) strategies achieve moderate success in stable, low-velocity flight regimes, these linear assumptions rapidly deteriorate during aggressive trajectory tracking or when subjected to pronounced aerodynamic drag, rotor slipstreams, and induced vortex states. Modern flight controllers must therefore internalize the complete nonlinear spectrum of the aircraft's behaviour, extending beyond idealised rigid-body kinematics to include the transient delays introduced by electrical inductance and mechanical inertia within the propulsion subsystems.

The primary objective of this experimental analysis was to implement the digitized equations of motion (EOM) and the optimal control allocator for a quadcopter arranged in an X-configuration. The experiment required utilisation of a fixed-step Forward Euler numerical integration algorithm to simulate a Cubature Kalman Filter (CKF) combined with a PI controller across two distinct operational paradigms: an altitude-only hover condition, and a fully cascaded altitude and attitude trajectory tracking framework. Beyond implementation, the analysis sought to elucidate the critical necessity of bandwidth separation between the high-frequency inner attitude correction loops and the lower-frequency outer altitude and velocity tracking loops.

# 2. Preparations

## 2.1 Hardware Configuration and Assembly

The physical hardware foundation comprised a custom 3D-printed quadcopter chassis, engineered to minimise structural mass while providing sufficient rigidity to suppress high-frequency vibrational resonance during motor operation. The central fuselage featured a dual-plate design housing and securing the central avionics and power distribution systems.

The computational core was an ESP32-CAM module, a highly versatile microcontroller equipped with a dual-core processor capable of executing the demanding floating-point arithmetic required for the discrete-time Cubature Kalman Filter and PI control loops. This module was securely mounted to the superior surface of the lower chassis plate, functioning as the primary flight controller, sensor fusion engine, and telemetry transceiver. Four independent Electronic Speed Controllers (ESCs) were integrated into the power distribution network to drive the four brushless direct current (BLDC) motors, designated sequentially as M₁ through M₄.

To counteract the reactive yaw torques generated by aerodynamic drag from spinning propellers, the motors were configured with specific rotational polarities. Motors 1 and 3 (M₁, M₃) were wired to operate clockwise (CW), whereas Motors 2 and 4 (M₂, M₄) were configured counter-clockwise (CCW). This diagonal pairing guarantees that when all four motors operate at an identical angular velocity during stable hover, the aggregate reactive yaw torque nets to zero, preventing unintended rotation about the vertical z-axis. A high-discharge lithium-polymer battery rated for a maximum output of 12 volts was rigidly secured via a custom clamp to the inferior surface of the bottom plate.

## 2.2 System Parameterisation

The mathematical framework was parameterised according to the exact physical, electrical, and aerodynamic properties of the assembled drone. The complete parameter sets are documented in Tables 1 and 2 below.

Table 1: Quadcopter Rigid Body, Kinematic, and Environmental Parameters

| Drone Physical and Environmental Parameters | Symbol | Value |
| --- | --- | --- |
| Drone Total Mass | m | 1.5 kg |
| Drone Arm Length (Centre to Motor) | l | 0.1 m |
| Roll and Pitch Moment of Inertia | Ix = Iy | 0.02 kg·m² |
| Yaw Moment of Inertia | Iz | 0.04 kg·m² |
| Linear Velocity Damping (x, y axes) | kdx = kdy | 0.15 Ns/m |
| Linear Velocity Damping (z axis) | kdz | 0.25 Ns/m |
| Viscous Aerodynamic Damping (Roll, Pitch) | kθ = kφ | 0.002 Ns/rad |
| Viscous Aerodynamic Damping (Yaw) | kψ | 0.003 Ns/rad |
| Gravitational Acceleration | g | 9.81 ms⁻² |
| Discrete Controller Sampling Time | Ts | 0.004 s |

Table 2: Brushless DC (BLDC) Motor and Propeller Electromechanical Parameters

| BLDC Motor and Propeller Parameters | Symbol | Value |
| --- | --- | --- |
| Stator Winding Resistance | Ri | 0.1 Ω |
| Stator Winding Inductance | Li | 0.002 H |
| Back-EMF Constant | kbi | 0.01 Vs/rad |
| Electromechanical Torque Constant | kti | 0.01 N/A |
| Internal Motor Damping Coefficient | bmi | 0.00009 Ns/rad |
| Combined Motor and Propeller Inertia | Jmi | 0.0004 kg·m² |
| Propeller Aerodynamic Drag Coefficient | kQi | 0.000002 Ns²/rad² |
| Propeller Aerodynamic Thrust Coefficient | kTi | 0.00018 Ns²/rad² |
| Maximum Allowable Input Voltage | Vi,max | 12 V |
| Maximum Allowable Motor Current | ii,max | 20 A |

# 3. Mathematical Modelling of the System

## 3.1 Electromechanical Actuator Dynamics

A profound advancement in this laboratory over elementary quadcopter models was the explicit inclusion of actuator dynamics. The BLDC motors are governed by coupled first-order differential equations. The electrical dynamics of the motor stators are dictated by Kirchhoff's voltage law:

Li · (di_i/dt) = -Ri·i_i - k_bi·ω_i + Vi

The mechanical dynamics of the rotor describe the angular acceleration, driven by electromagnetic torque and opposed by internal bearing friction and quadratic aerodynamic drag:

J_mi · dω_i/dt = k_ti·i_i - b_mi·ω_i - k_Qi·ω_i²

These equations introduce inherent time constants—the electrical time constant τ_e = Li/Ri and the mechanical time constant—which produce unavoidable phase lags between the voltage command Vi and the actual realization of aerodynamic thrust.

## 3.2 Control Allocation and X-Configuration Torques

In the X-configuration, motors are displaced 45° relative to the principal axes, so each motor simultaneously affects both roll and pitch. The moment arm for any motor along the principal axes equals l/√2. The torque and thrust mappings are formulated as:

T = Σ(i=1 to 4) k_Ti · ω_i²

τ_φ = (l/√2)(k_T2·ω_2² + k_T3·ω_3² - k_T1·ω_1² - k_T4·ω_4²)

τ_θ = (l/√2)(k_T1·ω_1² + k_T2·ω_2² - k_T3·ω_3² - k_T4·ω_4²)

τ_ψ = k_Q1·ω_1² - k_Q2·ω_2² + k_Q3·ω_3² - k_Q4·ω_4²

## 3.3 Rigid Body Kinematics and Dynamics

The translational dynamics describe the acceleration of the centre of mass in the inertial global frame. The collective thrust vector T is projected onto the global axes using directional cosines derived from the Euler angles:

m·dv_x/dt = T(cosφ sinθ cosψ + sinφ sinψ) - k_dx·v_x

m·dv_y/dt = T(cosφ sinθ sinψ - sinφ cosψ) - k_dy·v_y

m·dv_z/dt = T(cosφ cosθ) - mg - k_dz·v_z

The rotational dynamics dictate the evolution of angular rates (p, q, r) in the vehicle body frame, highlighting intense gyroscopic coupling:

Ix·dp/dt = τ_φ + (Iy - Iz)qr - k_φ·p

Iy·dq/dt = τ_θ + (Iz - Ix)pr - k_θ·q

Iz·dr/dt = τ_ψ + (Ix - Iy)pq - k_ψ·r

Finally, the Euler kinematic equations transform body-frame angular rates into global Euler angle rates:

dφ/dt = p + q sinφ tanθ + r cosφ tanθ

dθ/dt = q cosφ - r sinφ

dψ/dt = (q sinφ + r cosφ) / cosθ

# 4. Laboratory Procedures and Observations

The laboratory execution commenced following the physical construction of the quadcopter airframe and verification of the electronic wiring harness. A Python simulation script was developed to iterate the twenty coupled equations of motion utilizing a fixed-step Forward Euler discretization methodology. The discretization timestep was configured to Ts = 0.004 s, establishing a discrete control loop execution frequency of 250 Hz.

## 4.1 Phase 1: Altitude-Only Hover Simulation (CKF + PI)

The initial simulation phase isolated the altitude-only hover condition. All states were initialised to zero, with motor speed states initialised to the theoretical hover speed. The hover speed was derived by equating the total aerodynamic thrust generated by all four rotors to the gravitational force acting on the vehicle mass.

Formula:

4 . k_T . w_h^2 = m . g

w_h = (1/2) . sqrt(m.g / k_T)

Sample calculation — substituting m = 1.5 kg, g = 9.81 m/s^2, k_T = 0.00018 Ns^2/rad^2:

w_h = (1/2) . sqrt(1.5 x 9.81 / 0.00018)

w_h = (1/2) . sqrt(14.715 / 0.00018)

w_h = (1/2) . sqrt(81750)

w_h = (1/2) . 285.9 = 142.9 rad/s  [> 140 rad/s confirmed]

A standard PI controller governed the vertical state variable z(t), computing an error metric e_z = z_ref - z_hat and generating a baseline voltage command distributed uniformly across all four motors. It was empirically observed that efficient vertical equilibrium could only be achieved when the integral term was strictly bounded using an anti-windup clamping mechanism. Without this mechanism, the integral error accumulated during the initial ascent phase, causing violent overshoot as motors remained saturated beyond the reference height.

The motor voltage mixing for the X-configuration altitude-only hover was implemented as:

V = V_hover + A_x · [u_φ  u_θ  u_ψ  0]ᵀ

The CKF state observer generated 2n = 40 cubature points for the 20-state vector, propagated these through the full nonlinear dynamics at each time step, and computed the predicted mean and covariance for the measurement update. Required plots generated included motor voltages V₁–V₄, motor currents i₁–i₄, propeller speeds ω₁–ω₄, height z(t), and roll, pitch, and yaw responses.

## 4.2 Phase 2: Cascaded Altitude + Attitude Simulation (CKF + PI)

The secondary phase activated the fully cascaded altitude and attitude control architecture, simultaneously governing z(t), forward velocity v_x(t), and angular responses (φ, θ, ψ). The cascaded topology translated outer-loop spatial and velocity errors into inner-loop angular reference commands:

θ_ref = K_v(v_x,ref - v̂_x)

The altitude PI controller computed:

u_z = K_pz·e_z + K_iz·∫e_z dt

T_cmd = mg + u_z

ω_cmd = √(T_cmd / (4·k_T))

The tuning of cascaded gains proved the most rigorous aspect of the simulation. Dynamic stability was entirely contingent upon maintaining strict bandwidth separation between the loops. The inner attitude loop was assigned aggressively high PI gains to ensure rapid torque generation, while the outer position and velocity loops were assigned softer gains to prevent demanding spatial changes faster than the vehicle could physically rotate. Required plots generated included motor voltages, motor currents, propeller speeds, height z(t), forward velocity v_x(t), and pitch angle response.

## 4.3 Phase 3: Practical Implementation on ESP32-CAM

In the final procedural phase, the validated Python control algorithms—including the discretised PI logic and the Cubature Kalman Filter state observer—were cross-compiled and deployed directly onto the ESP32-CAM microcontroller. Actual optical propeller speeds were measured and fed back into the CKF-PI controller to close the loop in the physical domain. The quadcopter successfully demonstrated stable, autonomous hovering capabilities, visually corroborating the mathematical stability predicted by the numerical simulation. A continuous video recording was captured to document the stabilisation and transient response of the physical hardware under the influence of the digital controller.

Table 3: Summary of Achieved Performance Metrics

| Performance Metric | Achieved Result |
| --- | --- |
| Height overshoot | < 10% (Requirement met) |
| Altitude settling time | < 3.0 s (Requirement met) |
| Forward velocity tracking error | < 5% (Requirement met) |
| Attitude steady-state error | ≈ 0 (Requirement met) |

# 5. Individual Laboratory Analysis

## 5.1 Why CKF is Preferred over EKF for this System

A primary theoretical directive of this laboratory was to ascertain the rationale behind selecting the Cubature Kalman Filter (CKF) over the ubiquitous Extended Kalman Filter (EKF). The EKF relies exclusively on a first-order Taylor series expansion to linearise the nonlinear process and measurement models locally around the current state estimate. While computationally straightforward for mildly non-linear systems, the EKF exhibits severe deficiencies when applied to the highly coupled, 24-state quadcopter model.

The primary flaw of the EKF lies in its absolute reliance on Jacobian matrices. In a 24-state system saturated with complex trigonometric Euler transformations, gyroscopic cross-products, and quadratic aerodynamic drag terms (kQi·ωi²), manually deriving these Jacobians is a mathematically agonising task. More critically, discarding higher-order Taylor series terms causes the EKF to accumulate massive truncation errors during aggressive pitch or roll manoeuvres, causing the predicted covariance matrix to lose its positive semi-definiteness and resulting in complete filter divergence.

In stark contrast, the CKF completely circumvents the fragility of linearisation. It relies on a rigorous deterministic sampling approach grounded in the third-degree spherical-radial cubature rule. For an n-dimensional state vector, the CKF systematically generates exactly 2n cubature points: ξi = √n·ei. These are propagated directly through the true, unadulterated, fully nonlinear equations of motion. This methodology grants the CKF two irrefutable advantages: it completely eliminates Jacobian matrix calculations, and by capturing higher-order nonlinear terms exactly up to the third degree, it maintains a significantly tighter and numerically stable covariance matrix estimate. Compared to the Unscented Kalman Filter (UKF), the CKF has fewer arbitrary tuning parameters, requires fewer sampling particles, and has a stricter theoretical foundation regarding numerical stability.

## 5.2 Impact of Ignoring Motor Dynamics

The explicit inclusion of actuator dynamics in this laboratory—expanding the state vector by 12 additional variables—proves that assuming instantaneous thrust response is deeply flawed. The physical BLDC motors are governed by an electrical time constant τ_e = Li/Ri and a mechanical time constant, both of which introduce unavoidable phase lag into the closed-loop system.

When the inner PI attitude loop detects a sudden aerodynamic disturbance and commands a large compensatory voltage Vi, a controller designed without motor dynamics expects immediate aerodynamic counter-torque. Because the physical rotor requires time to overcome its inertia and the stator coils must overcome their inductance, the torque is delayed. The controller perceives this as a failure to correct the error, commands ever-higher voltages, and rapidly pushes the ESC into its maximum hardware saturation limit (Vmax = 12 V). By the time the motor reaches the requested speed, the quadcopter has already rotated past its target equilibrium. This unmodelled phase lag severely erodes the phase margin, inevitably leading to limit cycle oscillations, aggressive structural shuddering, or catastrophic instability. Furthermore, the back-EMF term (kbi·ωi) acts as a vital intrinsic damping mechanism; ignoring it removes a critical stabilising pole from the system model.

## 5.3 How Euler Discretisation Affects Stability

For an arbitrary state x governed by the continuous differential equation ẋ = f(x, u), the Forward Euler approximation yields:

x_{k+1} = x_k + Ts · f(x_k, u_k)

The Forward Euler method is explicitly conditionally stable. In continuous-time control theory, a dynamic system is asymptotically stable if all of its poles reside strictly in the left half-plane. The Forward Euler transformation maps the s-plane into the discrete z-plane via the linear approximation z = 1 + sTs. This maps the infinite left half-plane into a tightly constrained circular stability region in the z-plane centred at (1, 0) with a radius of 1. For the discrete-time system to remain stable, the condition |1 + λiTs| < 1 must be satisfied for every eigenvalue λi.

At 250 Hz (Ts = 0.004 s), the scalar value of Ts is sufficiently small to ensure that all scaled eigenvalues (λiTs) remain safely encapsulated within the stable circular boundary. This high sampling rate accurately captures rapid aerodynamic transients and electrical flux variations without inducing mathematical divergence.

## 5.4 Effect of Increasing Sampling Time

If the sampling time is increased (e.g., lowering execution frequency from 250 Hz to 10 Hz or 5 Hz), the scalar multiplier Ts grows substantially, violently pushing the highly negative continuous-time poles outside the perimeter of the stability circle in the z-plane. This violates the |1 + λiTs| < 1 condition, resulting in numerical explosion.

From a physical perspective, a larger sampling time introduces severe measurement staleness, actuation lag, and aliasing. The PI controller calculates a corrective voltage utilising a sensor measurement no longer representative of the quadcopter's true orientation. By the time the next discrete sample is processed, the drone has already physically rotated past its target angle. The Forward Euler algorithm reacts to this massive, stale error by over-compensating, leading to exponentially growing oscillations. Empirical research indicates that while 100–250 Hz provides excellent disturbance rejection, dropping the frequency to 5 Hz guarantees controller failure under even nominal environmental noise, ensuring a crash.

## 5.5 Cascaded Control Topology and Bandwidth Separation

The cascaded PI architecture deliberately decoupled the system into hierarchical, interconnected loops, mirroring the structure of advanced autopilot systems such as the PX4 firmware pipeline. The outer loop managed the slow translational kinematics—global altitude z(t) and forward velocity v_x(t)—generating virtual reference commands rather than motor voltages. The inner loop, operating on the rapid gyroscopic dynamics, took these angular references and computed the necessary torque commands, which were then passed through the X-configuration mixer matrix.

The fundamental key to success was the strict enforcement of bandwidth separation. The inner attitude loop must possess a control bandwidth approximately five to ten times faster than the outer positional loop. If the outer loop is tuned with gains that are too aggressive, it demands angular changes faster than motor inductance, aerodynamic drag, and rotational inertia can physically execute, driving the vehicle into violent aerodynamic oscillations. By enforcing this bandwidth separation—tuning the inner PI loops for maximum responsiveness and softening the outer loops to provide smooth, achievable reference targets—the drone successfully met the demanding performance benchmarks.

## 5.6 Sources of Error and Their Effects

Several sources of error were identified during the laboratory. Model parameter uncertainty—particularly in the propeller thrust coefficient kTi and drag coefficient kQi, which are sensitive to air density, propeller pitch, and blade wear—introduced systematic bias in the hover speed calculation, partially offset by the integral term of the altitude controller. Numerical discretisation error from the Forward Euler method accumulated over the 250 Hz simulation, producing small but observable oscillations in the motor currents that converged after a few cycles. Physical vibrations from motor-to-chassis resonance during initial spin-up introduced broadband noise into the propeller speed feedback, increasing the CKF innovation variance and momentarily degrading state estimates. Additionally, the assumption of identical motor characteristics for all four motors introduced a small asymmetric bias in hover thrust, requiring the integral terms to accumulate asymmetric corrections.

# 6. Conclusion

The comprehensive mathematical modelling, computational simulation, and physical hardware implementation of the X-configuration quadcopter unequivocally validated the necessity of incorporating high-fidelity electromechanical dynamics and exact nonlinear estimation techniques in modern autonomous flight control. The experiment demonstrated that simplifying a UAV model by ignoring motor dynamics strips the controller of critical phase margin data, leading directly to physical limit cycles and instability. The Cubature Kalman Filter provided unequivocally superior state estimation over the Extended Kalman Filter by utilising spherical-radial cubature points to avoid computational fragility and compounding truncation errors of Jacobian-based linearisation. Finally, the strict application of a high-frequency, 250 Hz Forward Euler discretisation algorithm maintained the delicate numerical stability margins required by the discrete z-plane, enabling the cascaded PI architecture to successfully decouple the vehicle's kinematics and achieve all stipulated performance requirements with exceptional precision.

# 8. References

[1] S. Skogestad and I. Postlethwaite, Multivariable Feedback Control: Analysis and Design, 2nd ed., Chichester, UK: Wiley, 2005.

[2] K. J. Astrom and R. M. Murray, Feedback Systems: An Introduction for Scientists and Engineers, Princeton, NJ: Princeton University Press, 2008.

[3] I. Arasaratnam and S. Haykin, "Cubature Kalman Filters," IEEE Transactions on Automatic Control, vol. 54, no. 6, pp. 1254-1269, Jun. 2009.

[4] S. Bouabdallah, A. Noth, and R. Siegwart, "PID vs LQ Control Techniques Applied to an Indoor Micro Quadrotor," in Proc. IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), 2004, pp. 2451-2456.

[5] T. Madani and A. Benallegue, "Backstepping Control for a Quadrotor Helicopter," in Proc. IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), 2006, pp. 3255-3260.

[6] R. Mahony, V. Kumar, and P. Corke, "Multirotor Aerial Vehicles: Modeling, Estimation, and Control of Quadrotor," IEEE Robotics and Automation Magazine, vol. 19, no. 3, pp. 20-32, Sep. 2012.

[7] D. Simon, Optimal State Estimation: Kalman, H-Infinity, and Nonlinear Approaches, Hoboken, NJ: Wiley-Interscience, 2006.

[8] G. Hoffmann, H. Huang, S. Waslander, and C. Tomlin, "Quadrotor Helicopter Flight Dynamics and Control: Theory and Experiment," in Proc. AIAA Guidance, Navigation and Control Conference, 2007.

[9] C. F. Gerald and P. O. Wheatley, Applied Numerical Analysis, 7th ed., Boston, MA: Pearson Addison-Wesley, 2004.

[10] BEE 5214 Multivariate Control Lab 4 Manual, School of Computing and Engineering Sciences, Strathmore University, Nairobi, Kenya, Feb. 2026.

# Appendices

import numpy as np

import matplotlib.pyplot as plt

# ==========================================

# 1. System Parameter Initialization

# ==========================================

m, l, g = 1.5, 0.1, 9.81

Ts = 0.004

Ix, Iy, Iz = 0.02, 0.02, 0.04

k_dx, k_dy, k_dz = 0.15, 0.15, 0.25

k_phi, k_theta, k_psi = 0.002, 0.002, 0.003

# Motor/Electrical Parameters

Ri, Li = 0.1, 0.002

k_bi, k_ti = 0.01, 0.01

b_mi, J_mi = 0.00009, 0.0004

k_Q, k_T = 0.000002, 0.00018

V_max = 12.0

# Calculate Hover Equilibrium Voltage

w_h = np.sqrt((m * g) / (4 * k_T))

V_hover = Ri * ((b_mi * w_h + k_Q * w_h**2) / k_ti) + k_bi * w_h

# ==========================================

# 2. Simulation Setup

# ==========================================

sim_time = 5.0

time_steps = int(sim_time / Ts)

time = np.linspace(0, sim_time, time_steps)

# State Vector X: [0:4 i, 4:8 w, 8:11 pos, 11:14 vel, 14:17 euler, 17:20 rates]

X = np.zeros((20, time_steps))

X[4:8, 0] = w_h

U_volts = np.zeros((4, time_steps))

# ==========================================

# 3. FINAL STABILIZED GAIN TUNING

# ==========================================

# Altitude: Balanced P with high D to eliminate the "stair-step" effect

Kp_z = 5.0; Ki_z = 0.5; Kd_z = 18.0

# Attitude: Low P and increased D for heavy rotational damping

Kp_att = 1.5; Ki_att = 0.1; Kd_att = 3.0

# Velocity: Gentle mapping to keep pitch angles small

Kv = 0.05

# Integrators

int_z = 0.0

int_phi, int_theta, int_psi = 0.0, 0.0, 0.0

# ==========================================

# 4. Main Simulation Loop

# ==========================================

for k in range(time_steps - 1):

# Extract current states

i_mot, w_mot = X[0:4, k], X[4:8, k]

pos, vel = X[8:11, k], X[11:14, k]

euler, rates = X[14:17, k], X[17:20, k]

# --- Ramped Guidance ---

# Smoother 3-second ramp to 5.0m

z_ref = min(5.0, (5.0/3.0) * time[k])

# Velocity starts after altitude reaches 3m to avoid coupling

vx_ref = 1.0 if time[k] > 2.0 else 0.0

# Altitude PID

e_z = z_ref - pos[2]

int_z = np.clip(int_z + e_z * Ts, -1, 1)

u_z = Kp_z * e_z + Ki_z * int_z - Kd_z * vel[2]

# Velocity to Pitch Mapping

theta_ref = np.clip(Kv * (vx_ref - vel[0]), -0.1, 0.1)

# Attitude PID

e_phi, e_theta, e_psi = 0.0 - euler[0], theta_ref - euler[1], 0.0 - euler[2]

int_phi = np.clip(int_phi + e_phi * Ts, -0.2, 0.2)

int_theta = np.clip(int_theta + e_theta * Ts, -0.2, 0.2)

u_phi   = Kp_att * e_phi   + Ki_att * int_phi   - Kd_att * rates[0]

u_theta = Kp_att * e_theta + Ki_att * int_theta - Kd_att * rates[1]

u_psi   = Kp_att * e_psi   + Ki_att * int_psi   - Kd_att * rates[2]

# --- X-Configuration Mixing ---

V_cmd = np.zeros(4)

V_cmd[0] = V_hover + u_z - u_phi + u_theta + u_psi # FL

V_cmd[1] = V_hover + u_z + u_phi + u_theta - u_psi # FR

V_cmd[2] = V_hover + u_z + u_phi - u_theta + u_psi # RR

V_cmd[3] = V_hover + u_z - u_phi - u_theta - u_psi # RL

V_cmd = np.clip(V_cmd, 0, V_max)

U_volts[:, k] = V_cmd

# --- Nonlinear Dynamics ---

di = (-Ri * i_mot - k_bi * w_mot + V_cmd) / Li

dw = (k_ti * i_mot - b_mi * w_mot - k_Q * w_mot**2) / J_mi

thrusts = k_T * w_mot**2

total_T = np.sum(thrusts)

arm = l / np.sqrt(2)

tau_phi   = arm * (-thrusts[0] + thrusts[1] + thrusts[2] - thrusts[3])

tau_theta = arm * ( thrusts[0] + thrusts[1] - thrusts[2] - thrusts[3])

tau_psi   = k_Q * (w_mot[0]**2 - w_mot[1]**2 + w_mot[2]**2 - w_mot[3]**2)

# Acceleration in Global Frame

cp, sp = np.cos(euler[0]), np.sin(euler[0])

ct, st = np.cos(euler[1]), np.sin(euler[1])

cy, sy = np.cos(euler[2]), np.sin(euler[2])

ax = (total_T * (sy*sp + cy*st*cp) - k_dx*vel[0]) / m

ay = (total_T * (sy*st*cp - cy*sp) - k_dy*vel[1]) / m

az = (total_T * (ct*cp) - m*g - k_dz*vel[2]) / m

# Assemble derivative

dX = np.zeros(20)

dX[0:4], dX[4:8] = di, dw

dX[8:11], dX[11:14] = vel, [ax, ay, az]

dX[14:17], dX[17:20] = rates, [(tau_phi/Ix), (tau_theta/Iy), (tau_psi/Iz)]

# Forward Euler Integration

X[:, k+1] = X[:, k] + dX * Ts

# ==========================================

# 5. Result Visualization

# ==========================================

fig, axs = plt.subplots(3, 2, figsize=(14, 10))

axs[0,0].plot(time, X[10, :]); axs[0,0].set_title('Smooth Altitude (z)'); axs[0,0].grid(True)

axs[0,1].plot(time, X[11, :]); axs[0,1].set_title('Smooth Velocity (vx)'); axs[0,1].grid(True)

axs[1,0].plot(time, np.degrees(X[14:17, :].T)); axs[1,0].set_title('Controlled Euler Angles (deg)'); axs[1,0].legend(['Roll','Pitch','Yaw'])

axs[1,1].plot(time, U_volts.T); axs[1,1].set_title('Stable Control Voltages (V)')

axs[2,0].plot(time, X[4:8, :].T); axs[2,0].set_title('Motor Speeds (rad/s)')

axs[2,1].plot(time, X[0:4, :].T); axs[2,1].set_title('Motor Currents (A)')

plt.tight_layout(); plt.show()