SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONIC ENGINEERING

Introduction to Vision and Robotics

Peter Corke’s Chapter 2 Lab

By

Edward Aribu – 142092

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%

%%

% Copyright 2022-2023 Peter Corke, Witold Jachimczyk, Remo Pillat

close all  % close all existing figures

%% 2.2 Working in Two Dimensions (2D)

% 2.2.1 Orientation in Two Dimensions

% 2.2.1.1 2D Rotation Matrix

R = rotm2d(0.3)

plottform2d(R);

det(R)

det(R*R)

syms theta real

R = rotm2d(theta)

simplify(R * R)

det(R)

simplify(ans)

output:

>> peter_corkes_chapter2_code

R =
0.9553 -0.2955
0.2955 0.9553

ans =
    1.0000
ans =
    1.0000
R =
[cos(theta), -sin(theta)]
[sin(theta),  cos(theta)]
ans =
[cos(2*theta), -sin(2*theta)]
[sin(2*theta),  cos(2*theta)]
ans =
cos(theta)^2 + sin(theta)^2
ans =
1

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%

%%

% Copyright 2022-2023 Peter Corke, Witold Jachimczyk, Remo Pillat

close all  % close all existing figures

% 2.2.1.2 Matrix Exponential for Rotation

R = rotm2d(0.3);

L = logm(R)

S = skew2vec(L)

X = vec2skew(2)

skew2vec(X)

expm(L)

expm(vec2skew(S))

output:

>> peter_corkes_chapter2_code

L =
         0   -0.3000
    0.3000         0
S =
    0.3000
X =
     0    -2
     2     0
ans =
     2
ans =
    0.9553   -0.2955
    0.2955    0.9553
ans =
    0.9553   -0.2955
    0.2955    0.9553

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%

% 2.2.2 Pose in Two Dimensions

% 2.2.2.1 2D Homogeneous Transformation Matrix

rotm2d(0.3)

tformr2d(0.3)

TA = trvec2tform([1 2])*tformr2d(deg2rad(30))

clf; axis([0 5 0 5]); hold on % new plot with both axes from 0 to 5

plottform2d(TA,frame="A",color="b");

T0 = trvec2tform([0 0]);

plottform2d(T0, frame="0",color="k");  % reference frame

TB = trvec2tform([2 1])

plottform2d(TB,frame="B",color="r");

TAB = TA*TB

plottform2d(TAB,frame="AB",color="g");

TBA = TB*TA;

plottform2d(TBA,frame="BA",color="c");

P = [3;2];  % column vector

plotpoint(P',"ko",label="P");

inv(TA)*[P;1]

h2e(ans')

homtrans(inv(TA),P')

output:

>> peter_corkes_chapter2_code

ans =
0.9553 -0.2955
0.2955 0.9553
ans =
0.9553 -0.2955 0
0.2955 0.9553 0
0 0 1.0000
TA =
0.8660 -0.5000 1.0000
0.5000 0.8660 2.0000
0 0 1.0000
TB =
1 0 2
0 1 1
0 0 1
TAB =
0.8660 -0.5000 2.2321
0.5000 0.8660 3.8660
0 0 1.0000
ans =
    1.7321
   -1.0000
    1.0000
ans =
    1.7321   -1.0000
ans =
    1.7321   -1.0000

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%

% 2.2.2.2 Rotating a Coordinate Frame

clf; axis([-5 4 -1 5]);

T0 = trvec2tform([0 0]);

plottform2d(T0,frame="0",color="k");  % reference frame

TX = trvec2tform([2 3]);

plottform2d(TX,frame="X",color="b");  % frame {X}

TR = tformr2d(2);

plottform2d(TR*TX,framelabel="RX",color="g");

plottform2d(TX*TR,framelabel="XR",color="g");

C = [3 2];

plotpoint(C,"ko",label="C");

TC = trvec2tform(C)*TR*trvec2tform(-C)

plottform2d(TC*TX,framelabel="XC",color="r");

output:

>> peter_corkes_chapter2_code
TC =
   -0.4161   -0.9093    6.0670
    0.9093   -0.4161    0.1044
         0         0    1.0000

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%

% 2.2.2.3 Matrix Exponential for Pose

L = logm(TC)

S = skewa2vec(L)

expm(vec2skewa(S))

X = vec2skewa([1 2 3])

skewa2vec(X)

output:

>> peter_corkes_chapter2_code
L =
         0   -2.0000    4.0000
    2.0000         0   -6.0000
         0         0         0
S =
    2.0000    4.0000   -6.0000
ans =
   -0.4161   -0.9093    6.0670
    0.9093   -0.4161    0.1044
         0         0    1.0000
X =
     0    -1     2
     1     0     3
     0     0     0
ans =
     1     2     3

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

%% 2.2.2.4 2D Twists

S = Twist2d.UnitRevolute(C)

expm(vec2skewa(2*S.compact))

S.tform(2)

S.pole

S = Twist2d.UnitPrismatic([0 1])

S.tform(2)

T = trvec2tform([3 4])*tformr2d(0.5)

S = Twist2d(T)

S.w

S.pole

S.tform()

Output:

>> peter_corkes_chapter2_code
S = 
( 1; 2  -3 )
ans =
   -0.4161   -0.9093    6.0670
    0.9093   -0.4161    0.1044
         0         0    1.0000
ans =
   -0.4161   -0.9093    6.0670
    0.9093   -0.4161    0.1044
         0         0    1.0000
ans =
     3     2
S = 
( 0; 0  1 )
ans =
     1     0     0
     0     1     2
     0     0     1
T =
    0.8776   -0.4794    3.0000
    0.4794    0.8776    4.0000
         0         0    1.0000
S = 
( 0.5; 3.9372  3.1663 )
ans =
    0.5000
ans =
   -3.1663    3.9372
ans =
    0.8776   -0.4794    3.0000
    0.4794    0.8776    4.0000
         0         0    1.0000

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3 Working in Three Dimensions (3D)

% 2.3.1 Orientation in Three Dimensions

% 2.3.1.1 3D Rotation Matrix

R = rotmx(pi/2)

clf; plottform(R);

clf; animtform(R)

clf; plottform(R,anaglyph="rc", axis=[-2 2 -2 2 -2 2])

clf; animtform(R,anaglyph="rc", axis=[-2 2 -2 2 -2 2]);

R = rotmx(pi/2)*rotmy(pi/2)

clf; plottform(R)

rotmy(pi/2)*rotmx(pi/2)

output:

>> peter_corkes_chapter2_code
R =
1 0 0
0 0 -1
0 1 0
R =
     0     0     1
     1     0     0
     0     1     0
ans =
     0     1     0
     0     0    -1
    -1     0     0

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.2 Three-Angle Representations

R = rotmz(0.1)*rotmy(-0.2)*rotmz(0.3)

R = eul2rotm([0.1 -0.2 0.3],"ZYZ")

gamma = rotm2eul(R,"ZYZ")

R = eul2rotm([0.1 0.2 0.3],"ZYZ")

gamma = rotm2eul(R,"ZYZ")

eul2rotm(gamma,"ZYZ")

R = eul2rotm([0.1 0 0.3],"ZYZ")

rotm2eul(R,"ZYZ")

R = eul2rotm([0.3 0.2 0.1],"ZYX")

gamma = rotm2eul(R,"ZYX")

R = eul2rotm([0.3 0.2 0.1],"XYZ")

gamma = rotm2eul(R,"XYZ")

tripleangleApp

output:

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.2 Three-Angle Representations (version 2)

R = rotmz(0.1)*rotmy(-0.2)*rotmz(0.3)

R = eul2rotm([0.1 -0.2 0.3],"ZYZ")

gamma = rotm2eul(R,"ZYZ")

R = eul2rotm([0.1 0.2 0.3],"ZYZ")

gamma = rotm2eul(R,"ZYZ")

eul2rotm(gamma,"ZYZ")

R = eul2rotm([0.1 0 0.3],"ZYZ")

rotm2eul(R,"ZYZ")

R = eul2rotm([0.3 0.2 0.1],"ZYX")

gamma = rotm2eul(R,"ZYX")

R = eul2rotm([0.3 0.2 0.1],"XYZ")

gamma = rotm2eul(R,"XYZ")

tripleangleApp;

pause(0.5); % Give the app a split second to fully render on screen

% Find all open figures, including hidden App Designer UIFigures

allFigs = findall(0, 'Type', 'figure');

% The most recently opened window is always the first one in this array

targetApp = allFigs(1);

% Force it to the center

movegui(targetApp, 'center');

output:

>> peter_corkes_chapter2_code
R =
    0.9021   -0.3836   -0.1977
    0.3875    0.9216   -0.0198
    0.1898   -0.0587    0.9801
R =
    0.9021   -0.3836   -0.1977
    0.3875    0.9216   -0.0198
    0.1898   -0.0587    0.9801
gamma =
    0.1000   -0.2000    0.3000
R =
    0.9021   -0.3836    0.1977
    0.3875    0.9216    0.0198
   -0.1898    0.0587    0.9801
gamma =
   -3.0416   -0.2000   -2.8416
ans =
    0.9021   -0.3836    0.1977
    0.3875    0.9216    0.0198
   -0.1898    0.0587    0.9801
R =
    0.9211   -0.3894         0
    0.3894    0.9211         0
         0         0    1.0000
ans =
         0         0    0.4000
R =
    0.9363   -0.2751    0.2184
    0.2896    0.9564   -0.0370
   -0.1987    0.0978    0.9752
gamma =
    0.3000    0.2000    0.1000
R =
    0.9752   -0.0978    0.1987
    0.1538    0.9447   -0.2896
   -0.1593    0.3130    0.9363
gamma =
    0.3000    0.2000    0.1000

reading STL models......
Supermarine Spitfire Mk VIII by Ed Morley @GRABCAD
Gimbal models by Peter Corke using OpenSCAD

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.4 Two-Vector Representation

a = [0 0 -1];

o = [1 1 0];

R = oa2rotm(o,a)

Output:

>> peter_corkes_chapter2_code

R =
   -0.7071    0.7071         0
    0.7071    0.7071         0
         0         0   -1.0000

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.5 Rotation about an Arbitrary Vector

R = eul2rotm([0.1 0.2 0.3]);

rotm2axang(R)

[x,e] = eig(R)

theta = angle(e(2,2))

R = axang2rotm([1 0 0 0.3])

Output:

>> peter_corkes_chapter2_code

ans =
    0.7900    0.5834    0.1886    0.3655
x =
  -0.7900 + 0.0000i   0.1073 + 0.4200i   0.1073 - 0.4200i
  -0.5834 + 0.0000i   0.0792 - 0.5688i   0.0792 + 0.5688i
  -0.1886 + 0.0000i  -0.6944 + 0.0000i  -0.6944 + 0.0000i
e =
   1.0000 + 0.0000i   0.0000 + 0.0000i   0.0000 + 0.0000i
   0.0000 + 0.0000i   0.9339 + 0.3574i   0.0000 + 0.0000i
   0.0000 + 0.0000i   0.0000 + 0.0000i   0.9339 - 0.3574i
theta =
    0.3655
R =
    1.0000         0         0
         0    0.9553   -0.2955
         0    0.2955    0.9553

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.6 Matrix Exponential for Rotation

R = rotmx(0.3)

L = logm(R)

S = skew2vec(L)

expm(L)

expm(vec2skew(S))

R = rotmx(0.3);

R = expm(0.3*vec2skew([1 0 0]));

X = vec2skew([1 2 3])

skew2vec(X)

output:

>> peter_corkes_chapter2_code

R =
    1.0000         0         0
         0    0.9553   -0.2955
         0    0.2955    0.9553
L =
         0         0         0
         0         0   -0.3000
         0    0.3000         0
S =
    0.3000         0         0
ans =
    1.0000         0         0
         0    0.9553   -0.2955
         0    0.2955    0.9553
ans =
    1.0000         0         0
         0    0.9553   -0.2955
         0    0.2955    0.9553
X =
     0    -3     2
     3     0    -1
    -2     1     0
ans =
     1     2     3

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.1.7 Unit Quaternions

q = quaternion(rotmx(0.3),"rotmat","point")

q = q*q;

q.conj

q*q.conj()

q.rotmat("point")

q.compact()

q.rotatepoint([0 1 0])

output:

>> peter_corkes_chapter2_code

q = 
  quaternion
     0.98877 + 0.14944i +       0j +       0k
ans = 
  quaternion
     0.95534 - 0.29552i +       0j +       0k
ans = 
  quaternion
     1 + 0i + 0j + 0k
ans =
    1.0000         0         0
         0    0.8253   -0.5646
         0    0.5646    0.8253
ans =
    0.9553    0.2955         0         0
ans =
         0    0.8253    0.5646

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.2 Pose in Three Dimensions

% 2.3.2.1 Homogeneous Transformation Matrix

T = trvec2tform([2 0 0])*tformrx(pi/2)*trvec2tform([0 1 0])

clf; plottform(T);

tform2rotm(T)

tform2trvec(T)

output:

>> peter_corkes_chapter2_code

T =
1 0 0 2
0 0 -1 0
0 1 0 1
0 0 0 1
ans =
     1     0     0
     0     0    -1
     0     1     0
ans =
     2     0     1

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.2 Pose in Three Dimensions

% 2.3.2.2 Matrix Exponential for Pose

T = trvec2tform([2 3 4])*tformrx(0.3)

L = logm(T)

S = skewa2vec(L)

expm(vec2skewa(S))

X = vec2skewa([1 2 3 4 5 6])

skewa2vec(X)

output:

>> peter_corkes_chapter2_code

T =
    1.0000         0         0    2.0000
         0    0.9553   -0.2955    3.0000
         0    0.2955    0.9553    4.0000
         0         0         0    1.0000
L =
         0         0         0    2.0000
         0         0   -0.3000    3.5775
         0    0.3000         0    3.5200
         0         0         0         0
S =
    0.3000         0         0    2.0000    3.5775    3.5200
ans =
    1.0000         0         0    2.0000
         0    0.9553   -0.2955    3.0000
         0    0.2955    0.9553    4.0000
         0         0         0    1.0000
X =
     0    -3     2     4
     3     0    -1     5
    -2     1     0     6
     0     0     0     0
ans =
     1     2     3     4     5     6

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.3.2 Pose in Three Dimensions

% 2.3.3.3 3D Twists

S = Twist.UnitRevolute([1 0 0],[0 0 0])

expm(0.3*S.skewa());

S.tform(0.3)

S = Twist.UnitRevolute([0 0 1], [2 3 2], 0.5);

X = trvec2tform([3 4 -4]);

clf; hold on

view(3)

for theta = [0:0.3:15]

plottform(S.tform(theta)*X,style="rgb",labelstyle="none",LineWidth=2)

end

hold off

L = S.line();

L.plot("k:",LineWidth=2);

S = Twist.UnitPrismatic([0 1 0])

S.tform(2)

T = trvec2tform([1 2 3])*eul2tform([0.3 0.4 0.5]);

S = Twist(T)

S.w

S.pole

S.pitch

S.theta

Output:

>> peter_corkes_chapter2_code
S = 
S = 
( 1 0 0; 0 0 0 )
ans =
1.0000 0 0 0
0 0.9553 -0.2955 0
0 0.2955 0.9553 0
0 0 0 1.0000
S = 
S = 
( 0 0 0; 0 1 0 )
ans =
1 0 0 0
0 1 0 2
0 0 1 0
0 0 0 1
S = 
S = 
( 0.42969  0.46143  0.19003; 0.53084  2.5512  2.7225 )
ans =
    0.4297    0.4614    0.1900
ans =
    1.1714   -1.6232    1.2927
ans =
    1.9226
ans =
    0.6585

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.4 Advanced Topics

% 2.4.5 Distance Between Orientations

q1 = quaternion(rotmx(pi/2),"rotmat","point");

q2 = quaternion(rotmz(-pi/2),"rotmat","point");

q1.dist(q2)

output:

>> peter_corkes_chapter2_code
ans =
    2.0944

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.4 Advanced Topics

% 2.4.6 Normalization

R = eye(3);

det(R)-1

for i = 1:100

R = R*eul2rotm([0.2 0.3 0.4]);

end

det(R) - 1

R = tformnorm(R);

det(R)-1

q = q.normalize();

output:

>> peter_corkes_chapter2_code
ans =
     0
ans =
   3.7748e-15
ans =
     0

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.4 Advanced Topics

% 2.4.8 More About Twists

S = Twist.UnitRevolute([1 0 0],[0 0 0])

S.compact()

S.v

S.w

S.skewa()

expm(0.3*S.skewa)

S.tform(0.3)

S2 = S*S

S2.printline(mode="axang", unit="rad")

line = S.line()

clf; axis([-2 2 -2 2 -2 2]); view(3); xlabel("x"); ylabel("y"); zlabel("z"); grid on

line.plot("k:",LineWidth=2);

T = trvec2tform([1 2 3])*eul2tform([0.3 0.4 0.5]);

S = Twist(T)

S/S.theta

S.unit();

S.tform(0)

S.tform(1)

S.tform(0.5)

Output:

>> peter_corkes_chapter2_code
S = 
S = 
( 1 0 0; 0 0 0 )
ans =
1 0 0 0 0 0
ans =
0 0 0
ans =
1 0 0
ans =
0 0 0 0
0 0 -1 0
0 1 0 0
0 0 0 0
ans =
1.0000 0 0 0
0 0.9553 -0.2955 0
0 0.2955 0.9553 0
0 0 0 1.0000
ans =
1.0000 0 0 0
0 0.9553 -0.2955 0
0 0.2955 0.9553 0
0 0 0 1.0000
S2 = 
S2 = 
( 2 0 0; 0 0 0 )
t = (0, 0, 0), R = (2rad | 1, 0, 0)
line = 
{ -0 -0 -0; 1 0 0}
S = 
S = 
( 0.42969  0.46143  0.19003; 0.53084  2.5512  2.7225 )
ans = 
ans = 
( 0.6525  0.70069  0.28857; 0.8061  3.8741  4.1342 
ans =
     1     0     0     0
     0     1     0     0
     0     0     1     0
     0     0     0     1
ans =
    0.8799   -0.0810    0.4682    1.0000
    0.2722    0.8936   -0.3570    2.0000
   -0.3894    0.4416    0.8083    3.0000
         0         0         0    1.0000
ans =
    0.9692   -0.0687    0.2367    0.3733
    0.1179    0.9727   -0.2001    1.1386
   -0.2165    0.2218    0.9508    1.4498
         0         0         0    1.0000

>>

%% Robotics, Vision & Control 3e: for MATLAB

%% Chapter 2: Representing position & orientation

% 2.5 MATLAB Classes for Pose and Rotation

T = trvec2tform([1 2 3])*tformrx(0.3)  % create SE(3) matrix as a MATLAB matrix

whos T

T = se3(rotmx(0.3),[1 2 3])  % create SE(3) matrix as an se3 object

whos T

T.tform()

whos ans

T.rotm()

T.trvec()

clf; plotTransforms(T,FrameLabel="A",FrameColor="b");

clf; plottform(T);

printtform(T);

P = [4 5 6];  % 3D point as a row vector

T.transform(P)

T.transform(P',IsCol=true)

T = rigidtform3d(rotmx(0.3),[1 2 3])  % create SE(3) matrix as a rigidtform3d object

whos T

T.A

T.R

T.Translation

T.transformPointsForward(P)

q = quaternion(rotmx(0.3),"rotmat","point")

conj(q)

q.conj()

q.conj

output:

>> peter_corkes_chapter2_code
T =
1.0000 0 0 1.0000
0 0.9553 -0.2955 2.0000
0 0.2955 0.9553 3.0000
0 0 0 1.0000
Name Size Bytes Class Attributes
T 4x4 128 double 
T = 
se3
1.0000 0 0 1.0000
0 0.9553 -0.2955 2.0000
0 0.2955 0.9553 3.0000
0 0 0 1.0000
Name Size Bytes Class Attributes
T 1x1 136 se3 
ans =
1.0000 0 0 1.0000
0 0.9553 -0.2955 2.0000
0 0.2955 0.9553 3.0000
0 0 0 1.0000
Name Size Bytes Class Attributes
ans 4x4 128 double 
ans =
1.0000 0 0
0 0.9553 -0.2955
0 0.2955 0.9553
ans =
1 2 3

t = (1, 2, 3), RPY/zyx = (0, 0, 0.3) rad
ans =
5.0000 5.0036 10.2096
ans =
    5.0000
    5.0036
   10.2096
T = 
  rigidtform3d with properties:
    Dimensionality: 3
       Translation: [1 2 3]
                 R: [3×3 double]
                 A: [1.0000         0         0    1.0000
                          0    0.9553   -0.2955    2.0000
                          0    0.2955    0.9553    3.0000
                          0         0         0    1.0000]
  Name      Size            Bytes  Class           Attributes
  T         1x1                97  rigidtform3d              
ans =
    1.0000         0         0    1.0000
         0    0.9553   -0.2955    2.0000
         0    0.2955    0.9553    3.0000
         0         0         0    1.0000
ans =
    1.0000         0         0
         0    0.9553   -0.2955
         0    0.2955    0.9553
ans =
     1     2     3
ans =
    5.0000    5.0036   10.2096
q = 
  quaternion
     0.98877 + 0.14944i +       0j +       0k
ans = 
  quaternion
     0.98877 - 0.14944i +       0j +       0k
ans = 
  quaternion
     0.98877 - 0.14944i +       0j +       0k
ans = 
  quaternion
     0.98877 - 0.14944i +       0j +       0k

>>