SCHOOL OF COMPUTING AND ENGINEERING SCIENCES (SCES)

BACHELOR OF SCIENCE IN ELECTRICAL AND ELECTRONICS ENGINEERING (BSEEE)

PHYSICS EXPERIMENT ON COPLANAR FORCES AND MOTION

By

Mwongela Edward Aribu    142092

Collaborators

Teddy Mbugua - 142776

Mark Mbiriri - 143389

Arnold Injeni - 146279

# Abstract

This is a report that contains the data obtained in a laboratory experiment conducted in the Strathmore engineering laboratories by the group members listed above. It was conducted to investigate coplanar forces, moments of forces, kinetic friction, and the facts or theories behind or related to them as well as experiments on motion.

Table of Contents

Abstract	2

Table of figures	6

INTRODUCTION	8

moment	8

friction	8

Coplanar forces	8

LITERATURE REVIEW	9

Moment of a force	9

Friction	9

Composition of concurrent forces	9

The Elements of Coplanar Force Resolution	10

Hooke’s law	11

Coefficient of kinetic friction	11

PART 1	12

EXPERIMENT 1	12

METHODOLOGY	12

Materials required	12

Procedure	12

RESULTS AND FINDINGS	13

EXPERIMENT 2	17

METHODOLOGY	17

Material required	17

Procedure	17

RESULTS AND FINDINGS	17

Conclusion	18

EXPERIMENT 3	19

METHODOLOGY	19

Materials required	19

Procedure	19

RESULTS AND FINDINGS	19

Conclusions	21

EXPERIMENT 4	22

METHODOLOGY	22

Materials required	22

Procedure	22

RESULTS AND FINDINGS	22

Conclusions	24

EXPERIMENT 5	25

METHODOLOGY	25

Materials required	25

Procedure	25

RESULTS AND FINDINGS	25

EXPERIMENT 8	32

METHODOLOGY	32

Materials required	32

Procedure	32

RESULTS AND FINDINGS	32

Conclusions	32

EXPERIMENT 12	33

METHODOLOGY	33

Materials required	33

Procedure	33

RESULTS AND FINDINGS	33

EXPERIMENT 13	35

METHODOLOGY	35

Materials required	35

Procedure	35

RESULTS AND FINDINGS	35

Measurement of the coefficient of kinetic friction	41

METHODOLOGY	41

RESULTS AND FINDINGS	41

PART 2	44

Experiment 1	44

METHODOLOGY	44

RESULTS AND FINDINGS	44

Experiment 2	45

METHODOLOGY	45

RESULTS AND FINDINGS	46

Measurement of the elastic constant of a spring	47

Objective	47

METHODOLOGY	47

Required tools and materials	47

Phases of the experiment	47

RESULTS AND FINDINGS	48

Experiment 3	49

METHODOLOGY	49

Procedure	49

RESULTS AND FINDINGS	49

Experiment 4	51

METHODOLOGY	51

procedure	51

RESULTS AND FINDINGS	52

REFERENCES	54

# Table of figures

Figure 1	13

Figure 2	14

Figure 3	15

Figure 4	15

Figure 5	16

Figure 6	17

Figure 7	18

Figure 8	20

Figure 9	20

Figure 10	21

Figure 11	22

Figure 12	23

Figure 13	23

Figure 14	24

Figure 15	26

Figure 16	27

Figure 17	28

Figure 18	29

Figure 19	30

Figure 20	30

Figure 21	31

Figure 22	32

Figure 23	33

Figure 24	34

Figure 25	34

Figure 26	36

Figure 27	37

Figure 28	38

Figure 29	38

Figure 30	39

Figure 31	39

Figure 32	40

Figure 33	41

Figure 34	42

Figure 35	42

Figure 36	43

Figure 37	44

Figure 38	46

Figure 39	46

Figure 40	48

Figure 41	48

Figure 42	48

Figure 43	49

Figure 44	49

Figure 45	52

Figure 46	53

Figure 47	53

# INTRODUCTION

## moment

The moment of a force - is a measure of its tendency to cause a body to rotate about a specific point or axis. This is different from the tendency for a body to move, or translate, in the direction of the force. For a moment to develop, the force must act upon the body in such a manner that the body would begin to twist. This occurs every time a force is applied so that it does not pass through the centroid of the body. A moment is due to a force not having an equal and opposite force directly along its line of action.

(What Is a Moment?, n.d.)

Moment = Force x Distance or M = (F)(d)

## friction

Friction is defined as the resistance offered by the surfaces that are in contact when they move past each other. There are four types of friction, and they are classified as follows:

- Static friction

- Sliding friction

- Rolling friction

- Fluid friction

## Coplanar forces

Coplanar forces are forces on a single plane. This means that all points of application are inside that plane and that all forces are running parallel to that plane. Coplanar forces may be concurrent, non-concurrent, parallel, or non-parallel.

# LITERATURE REVIEW

## Moment of a force

In physics and mechanics, torque is the rotational equivalent of linear force. It is also referred to as the moment, moment of force, rotational force or turning effect, depending on the field of study.

The moment of a force or torque is defined as the turning effect of the force about a pivot and is the product of the force (F) and the perpendicular distance (d) from the line of action of the force to the pivot.

SI unit of moment of a force is Newton-meter (Nm). It is a vector quantity.

Its direction is given by the right-hand grip rule perpendicular to the plane of the force and pivot point which is parallel to the axis of rotation.

## Friction

Friction is caused due to the irregularities on the two surfaces in contact. So, when one object moves over the other, these irregularities on the surface get entangled, giving rise to friction. The more the roughness, the more irregularities and more significant will be the friction.(Friction - Definition, Types of Friction, Applications & Videos, n.d.)

Friction is a force that is dependent on external factors. Following are the two factors on which friction depends:

1. On the nature of the two surfaces that are in contact

Friction is dependent on the smoothness or roughness of the two surfaces that are in contact with each other. When the surface is smooth, the friction between the two reduces as there is not much interlocking of irregularities. While the surface is rough, friction increases.

2. On the force that is acting on these surfaces

Friction increases when the force is applied along with the irregularities.

## Composition of concurrent forces

Concurrent forces refer to those forces which intersect through a common point. When it comes to concurrent forces, we can add them together as vectors and this gives the net resultant.

There are two major types of equilibrium of concurrent forces. These include Static equilibrium and dynamic equilibrium. In static equilibrium, the state of the body is such that all the resultant forces add up to zero. The acceleration and velocity are both zero.

In dynamic equilibrium, the net result is zero, however, velocity is not. For example, a block resting on the floor can be said to be in static equilibrium. A block attached to a string in Simple Harmonic motion can be an example of dynamic equilibrium.

This is the simple way to define the Types of equilibrium of concurrent forces.

## The Elements of Coplanar Force Resolution

There are many ways in which forces can be manipulated. It is often easier to work with a large, complicated system of forces by reducing it to an ever-decreasing number of smaller problems. This is called the "resolution" of forces or force systems. This is one way to simplify what may otherwise seem to be an impossible system of forces acting on a body. Certain systems of forces are easier to resolve than others. Coplanar force systems have all the forces acting in in one plane. They may be concurrent, parallel, non-concurrent or non-parallel. All these systems can be resolved by using graphic statics or algebra.

A concurrent coplanar force system is a system of two or more forces whose lines of action ALL intersect at a common point. However, all the individual vectors might not actually be in contact with the common point. These are the simplest force systems to resolve with any one of many graphical or algebraic options.

A parallel coplanar force system consists of two or more forces whose lines of action are ALL parallel. This is commonly the situation when simple beams are analysed under gravity loads. These can be solved graphically but are combined most easily using algebraic methods.

The last illustration is of a non-concurrent and non-parallel system. This consists of several vectors that do not meet at a single point and none of them are parallel. These systems are essentially a jumble of forces and take considerable care to resolve.(Coplanar Force Resolution, n.d.)

## Hooke’s law

Stress and strain take different forms in different situations. Generally, for small deformations, the stress and strain are proportional to each other, and this is known as Hooke’s Law.

Hooke’s law states that the strain of the material is proportional to the applied stress within the elastic limit of that material.

When the elastic materials are stretched, the atoms and molecules deform until stress is applied, and when the stress is removed, they return to their initial state.

Mathematically, Hooke’s law is expressed as:

F = ke

In the equation is the force, x is the extension length, k is the constant of proportionality known as spring constant in N/m.

## Coefficient of kinetic friction

Different types of coefficients

The different types of friction are: Static, kinetic, deformation, molecular and rolling. Each has its own coefficient of friction.

Static coefficient: is the force that holds back a stationary object up to the point that it just starts moving. Thus, the static coefficient of friction concerns the force restricting the movement of an object that is stationary on a relatively smooth, hard surface.

Kinetic coefficient: Once you overcome static friction, kinetic friction is the force holding back regular motion. This, kinetic fiction coefficient of friction concerns the force restricting the movement of an object that is sliding on a relatively smooth, hard surface.

Deformation coefficient: The deformation coefficient of friction concerns the force restricting the movement of an object that is sliding or rolling and one or both surfaces are relatively soft and deformed by the forces.

Molecular coefficient of friction: Concerns the force restricting the movement of an object that is sliding on an extremely smooth surface or where a fluid is involved.

The rolling coefficient of friction: Combines static, deformation and molecular coefficients of friction. This coefficient of friction can be made quite low(Determining the Coefficient of Friction - Succeed in Physical Science, n.d.)

# PART 1

# EXPERIMENT 1

## METHODOLOGY

### Materials required

- 1 support for dynamometer

- 2 fixed pulleys

- 1 dynamometer 250gr (2.5N)

- 1 goniometer with magnetic support

- 2 series of 10gr weights with pan

- 1 cord

### Procedure

- Mount the dynamometer

- Mount the two pans loaded with weights chosen at will. The intensity of F1 and F2 is represented by the sum of the weights and of the respective pan. Pass the cord through the ring of the dynamometer and the groove of the pulleys.

- Make sure that the system is in equilibrium

- Fix the goniometer so that its centre coincides with the ring of the dynamometer.

- Measure the angle between the two forces. Align the zero of the goniometers with one of the two branches of the cord.

- Take note of the intensity of the resultant(R) on the dynamometer.

- Take note of the angle between F1 and R

- Repeat the experience, changing the values of F1, F2 and the angle

- Compare the force value on the dynamometer with that found graphically and analytically.

- By graphics, after having taken note of the values for the forces and of the angles, the relative vector diagram is constructed in scale of the graph paper. The graphical value of R, within the limit of experimental errors, must coincide with the value read on the dynamometer.

## RESULTS AND FINDINGS

Figure 1

Figure 2

Figure 3

Figure 4

| F1 | F2 | total | R | Angle(x0) |
| --- | --- | --- | --- | --- |
| 10 | 10 | 20 | 0.05 | 140 |
| 20 | 20 | 40 | 0.10 | 126 |
| 30 | 30 | 60 | 0.20 | 119 |
| 40 | 40 | 80 | 0.30 | 110 |
| 50 | 50 | 100 | 0.40 | 106 |

Figure 5

# EXPERIMENT 2

## METHODOLOGY

### Material required

- 1 support for dynamometer

- 1 dynamometer 250gr (2.5N)

- 1 rod for levers

- 1 S-shaped hook

- 2 series of 10gr weights with pan

- 1 cord

### Procedure

- Fix the dynamometer to the magnetic whiteboard with specific support

- Suspend the rod for levers on the dynamometer using the S-shaped hook and inserting it in the rod’s centre of mass

- Take note of the weights of the rod indicated on the dynamometer

- Apply the pans with the relative weight in the holes of the rod, on the opposite side with respect to the centre of the mass. Change the amount of weight on the pans and the distances from the centre of mass to balance the system

- When the system is in equilibrium, read the value of the resultant on the dynamometer, considering the initial tare represented by the weight of the rod.

- Repeat the experience changing the values of F1, F2, a and b.

## RESULTS AND FINDINGS

Figure 6

Weight of the rod = 0.5

| F2(grams) | F1(grams) | a(cm) | b | L | Total weight |
| --- | --- | --- | --- | --- | --- |
| 20 | 20 | 5 | 5 | 10 | 0.9 |
| 20 | 30 | 7 | 5 | 12 | 1.05 |
| 40 | 30 | 6.5 | 5 | 11.5 | 1.25 |
| 50 | 40 | 6.2 | 5 | 11.2 | 1.45 |
| 50 | 20 | 11.5 | 5 | 16.5 | 1.25 |
| 40 | 20 | 9 | 5 | 14 | 1.15 |

Figure 7

## Conclusion

- The resultant of two parallel and concurrent coplanar forces F1 and F2, is a force where the component forces have the same direction and orientation, and the intensity is equal to the sum of the intensities

- The point of application of the resultant divides the segment L in two parts a and b so that the following relation is always verified

F1 X a = F2 X b

# EXPERIMENT 3

## METHODOLOGY

### Materials required

- 1 support for dynamometer

- 1 dynamometer 250gr (2.5N)

- 1 rod for levers

- 1 S-shaped hook

- 2 series of 10gr weights with pan

- 1 fixed pulley

- 1 chord

### Procedure

- Repeat the previous operations 1-2-3

- Tie on end of the cord to the last hole of the rod

- Fasten the fixed pulley

- Apply a pan to one end of the cord and load it with weights, passing the cord through the groove of the pulley

- Moving the fixed pulley, make the cord perpendicular to the rod

- Find a combination of values F1, F2, b by which the system remains in equilibrium

- Take note of the value of the resultant, considering that the tare is added to the value of F2

- Repeat the test for different combinations of F1, F2, b as in the figure

## RESULTS AND FINDINGS

Figure 8

Figure 9

| F1 | F2 | a | b | Resultant force |
| --- | --- | --- | --- | --- |
| 60 | 70 | 17.5 | 15 | 0.60 |
| 20 | 30 | 17.5 | 12.5 | 0.50 |
| 40 | 70 | 17.5 | 10.5 | 0.75 |
| 10 | 20 | 17.5 | 9 | 0.60 |
| 10 | 30 | 17.5 | 6 | 0.70 |
| 20 | 40 | 17.5 | 9 | 0.70 |

Figure 10

## Conclusions

- The resultant of two discordant parallel forces is a force which has the same direction as the component forces, the orientation being that of the force of higher intensity and intensity being the difference of the intensities of the component forces.

- Its point of application falls outside on the straight line joining the points of application of the two forces and at such a distance that the following relation is verified.

# EXPERIMENT 4

## METHODOLOGY

### Materials required

- 1 support for dynamometer

- 1 dynamometer 250gr (2.5N)

- 2 fixed pulleys

- 2 series of 10gr weights with pan

- 1 cord

### Procedure

- Hook the cord to the dynamometer so that it can slide freely

- Fasten the pulleys to the whiteboard

- Apply the pans at the ends of the cord, loading them with some weights

- Acting on the pulleys and on the dynamometer, make the two branches of the cord perpendicular to each other

- Having balanced the system, take note of the force read on the dynamometer

- Repeat the experience, changing the values of Fx and Fy

- Trace the vector diagram and verify that the intensity of the force F read on the dynamometer is equal to Fx2+Fy2

## RESULTS AND FINDINGS

| Mass(g) | Fx 2 | Fy2 | F |
| --- | --- | --- | --- |
| 40 | 20 | 20 | 0.10 |
| 60 | 30 | 30 | 0.20 |
| 70 | 40 | 30 | 0.30 |
| 90 | 50 | 40 | 0.45 |
| 110 | 60 | 50 | 0.55 |

Figure 11

Figure 12

Figure 13

Figure 14

## Conclusions

- The forces Fx and Fy represent the components of the force F along the X and Y axes

# EXPERIMENT 5

## METHODOLOGY

### Materials required

- 1 magnetic support with a hook

- 1 spring with index

- 1 series of 10 gr weights

- 1 ruler

### Procedure

- Fasten the magnetic support with the hook to the whiteboard

- Suspend the spring to the hook without any load

- Take note of the height indicated by the arrow (ho) using the ruler

- Load the spring with the 10gr pan and take note of the new value (h1)

- Calculate the elongation ẟh = h1-h0

- Add a further weight of 10gr to the pan. Take note of the new value h2 and of the relative elongation ẟh = h2-h0

- Repeat the operation, adding 10gr weights each time

- Write the results in the table

- Trace a graph, making the applied weight of force the X-axis and the elongations the Y-axis

## RESULTS AND FINDINGS

Figure 15

Figure 16

Figure 17

Figure 18

Figure 19

ho = 63.5

| F | h1 | Dh | F/dh |
| --- | --- | --- | --- |
|  | 62 | 1.5 | 6.67 |
| 20 | 59.4 | 4.1 | 4.88 |
| 30 | 57.3 | 6.2 | 4.84 |
| 40 | 55.3 | 8.2 | 4.88 |
| 50 | 53.3 | 10.2 | 4.90 |
| 60 | 51.3 | 12.2 | 4.92 |

Figure 20

Figure 21

# EXPERIMENT 8

## METHODOLOGY

### Materials required

- 1 support for dynamometer

- 1 dynamometer 250gr (2.5N)

- 1 fixed pulley

- 1 moment disk

- 1 series of 10gr weights with pan

### Procedure

- Fix the moment disk

- Fix the dynamometer to the relative support

- Fasten the fixed pulley

- Hook the pans with some weights to the moment disk with the cords in the specific holes

- Hook the dynamometer to the moment disk with a cord, passing it through the groove of the fixed pulley

- Reach equilibrium and read the value of the balancing force in the dynamometer

- Repeat the test changing the application holes of the pans, the load on them and the point of application of the dynamometer

## RESULTS AND FINDINGS

| F1 | F2 | a | b | Balancing force |
| --- | --- | --- | --- | --- |
| 60 | 50 | 12.8 | 8.6 | 0.80 |
| 60 | 10 | 12.8 | 11.7 | 0.70 |
| 30 | 20 | 13.7 | 8.5 | 0.85 |
| 30 | 40 | 10.6 | 8.4 | 0.80 |

Figure 22

## Conclusions

-If clockwise moments are assigned a positive sign and anticlockwise negative, check that the following relation is met:

F3 · b3 – F1 · b1 – F2 · b2 = 0

# EXPERIMENT 12

## METHODOLOGY

### Materials required

- Inclined plane on magnetic bases with goniometer

- Magnetic support for dynamometer

- Dynamometer 250gr (2.5N)

- 1 fixed pulley]1 cord

- 1 carriage

- 1 series of 10gr weights

### Procedure

- Fix the inclined plane to the magnetic whiteboard

- Fix the support and the dynamometer

- Mount the fixed pulley

- Hook the carriage at one end of the cord

- Hook the other end of the cord to the dynamometer

- Make sure that the cord is parallel to the inclined plane

- Take note of the force indicated on the dynamometer

- Repeat the operation changing the inclination and/or load on the carriage

- Fill out the table

## RESULTS AND FINDINGS

Figure 23

Figure 24

Weight of carriage = 0.40N

| Inclination | Weight on carriage | Force on dynamometer |
| --- | --- | --- |
| 100 | 0.40 |  |
|  | 0.80 | 0.05 |
|  | 1.10 | 0.10 |
| 200 | 0.40 | 0.075 |
|  | 0.70 | 0.15 |
|  | 0.50 | 0.10 |
|  | 0.90 | 0.20 |
| 300 | 0.40 | 0.15 |
|  | 0.60 | 0.20 |
|  | 0.70 | 0.25 |
| 350 | 0.40 | 0.15 |
|  | 0.50 | 0.20 |
|  | 0.60 | 0.30 |
|  | 0.70 | 0.35 |

Figure 25

# EXPERIMENT 13

## METHODOLOGY

## Materials required

- 1 wooden block

- 1 series of 10gr weights

- 1 inclined plane with goniometer

- 1 fixed pulley

- 1 cord

- 1 dynamometer

### Procedure

- Weigh the wooden block with the dynamometer

- Fix the inclined plane in the horizontal position

- Hook the block to the cord

- Hook the pan to one end of the cord

- Low the pan until the wooden block starts to move

- Repeat the test loading the block with additional weight

## RESULTS AND FINDINGS

Figure 26

Figure 27

Figure 28

Figure 29

Figure 30

Figure 31

Figure 32

# Measurement of the coefficient of kinetic friction

## METHODOLOGY

- Place the wooden horizontal on the plane, slowly incline the plane until the block starts to slide (incipient motion). In this position, take note of the angle indicated on the goniometer. As the force of friction is proportional to the pressure force on the plane, in this case we have:

- Fa = Kr · N

- The graph shows us that

- N = P· cos *

- While in balance

- Fa = P· sin *

- The coefficient of friction is therefore expressed as

FaN=P·sin*P·cos*=tg*

## RESULTS AND FINDINGS

Figure 33

Figure 34

Figure 35

Weight of block = 0.35N

| Weight of block | angle | Mass on pan(grams) |
| --- | --- | --- |
| 0.55N | 100 | 30 |
| 0.55N | 200 | 40 |

Figure 36

# PART 2

# Experiment 1

## METHODOLOGY

- Set up the track using 2 pairs of photogates at distances s1, s2 from the lest end and mutual distance s2-s1 between them.

- Mount the included shading sheet on the cart (Δx is the shading sheet width)

- Connect the pair of photogates to the data logger at the digital input 1 and 2 and place the cart at the lest end of the track.

- Select the exercise number (1) in the software and enter the distances s1: first photogate distance from the left end, s2: second photogate distance from the left end.

- Press the START button on the software and push the cart with your hand in such a way it passes on the metal track in front of the two photogates.

- At the end of the run, the software detects the time t between the shading of the first photogate pair and the shading of the second photogate pair and the shading interval Δt1 and Δt2 of the individual photogate pairs.

- The software shows the speed values.

## RESULTS AND FINDINGS

| T1(ms) | dT | T2(ms) | dT |
| --- | --- | --- | --- |
| 269.285 | 81.457 | 1301.73 | 119.012 |
| 50.713 | 37.892 | 534.136 | 41.131 |
| 22.377 | 49.68 | 723.637 | 56.705 |

Figure 37

# Experiment 2

## METHODOLOGY

- Incline the track by certain angle.

- Set up the track using 2 pairs of photogates at distance s1, s2 from the lest end and mutual distance s2-s1 between them.

- Mount the included shading sheet on the cart (Δx is the shading sheet width)

- Connect the pair of photogates to the datalogger (digital inputs 1 and 2) and place the cart at the left end of the track holding it with one hand.

- Press the START button on the software and let the cart go.

- At the end of the run, detect through the software the time t between the shading of the first photogate pair and the shading of the second photogate pair and the shading interval Δt1 – Δt2 of the individual photogate pairs.

- The software will calculate the instantaneous acceleration as the shading passes in front of each pair of photogates. Hence it will be possible to verify that a is constant throughout the track and is equal:

A = g · sin *

- Where g is the acceleration of gravity and a is the inclination of the track with respect to the support surface.

- The acceleration at the 2 pairs of photogates is calculated as the ratio between the instantaneous speed and the passing time of the cart in front of the single photogate:

v1ΔxΔt1

v2ΔxΔt2

Δx is the width of the shading sheet.

Δt1 , Δt2 are the time intervals in which the two photogates are shaded.

## RESULTS AND FINDINGS

Figure 38

Angle of inclination = 40

Δx = 3.5cm

Distance between S1 and S2 = 51cm

| T1(ms) | dt1(ms) | T2(ms) | dt2(ms) |
| --- | --- | --- | --- |
| 3122.23 | 90.952 | 758.305 | 40.833 |
| 1231.43 | 93.826 | 762.539 | 41.509 |
| 242.79 | 70.988 | 577.372 | 31.729 |

Figure 39

V1=0.00385m/s

v2=0.00857m/s

# Measurement of the elastic constant of a spring

## Objective

- To compare the deformation of a spring with the intensity of the force applied to it, obtaining Hook’s Law, which links the force and the deformation of an elastic body.

## METHODOLOGY

### Required tools and materials

- Track

- Spiral spring

- Cart

- Masses

- Metric rod

### Phases of the experiment

- Hook the spring on one side to the end of the track, on the other side of the cart after weighing it.

- Incline the track by a certain angle.

- The spring under the action of the cart weight tends to stretch

- Determine the extension of the spring by means of a metric rod.

- Change the weight of the cart by adding gradually- increasing mass and immediately determine the elongation.

- Enter the data in the table (masses in the first column and elongations in the third column).

- Determine for each line the force exerted by the mass on the spring, multiplying the value of m by the acceleration of gravity (g = 9.81 m/s2) and by the sine of the track angle of inclination.

- Determine for each line the k between the force F (col. 2) and the stretching ΔL (col. 3)

- Compare the k- values shown in column 4

The same experiment can be carried out for the springs with different elastic constants.

## RESULTS AND FINDINGS

Figure 40

Weight of cart = 0.65N

| Mass (Kg) | F = mgsinɵ | ΔL | K(N/m) |
| --- | --- | --- | --- |
| 0.065 | 0.0698 | 0.72 | 0.0969 |
| 0.115 | 0.0787 | 3.2 | 0.0250 |
| 0.165 | 0.113 | 4.5 | 0.0251 |
| 0.215 | 0.147 | 6.2 | 0.0237 |

Figure 41

| Mass (Kg) | F = mgsinɵ | ΔL | K(N/m) |
| --- | --- | --- | --- |
| 0.065 | 0.0698 | 1 | 0.0698 |
| 0.115 | 0.0787 | 1.8 | 0.0437 |
| 0.165 | 0.113 | 2.5 | 0.0452 |
| 0215 | 0.147 | 3.4 | 0.0432 |

Figure 42

The constant for spring 1 is 0.025N/M

The constant for spring 2 is 0.044N/M

# Experiment 3

## METHODOLOGY

### Procedure

- Set up the track using 2 pairs of photogates at distances s1, s2 from the left end and mutual distance s2-s1 between them.

- Tie a wire to the end of the cart and connect it through a pulley to the weight hanger with weights.

- The shading sheet must be mounted on the cart.

- Connect the photogates to the datalogger (digit input 1 and 2) and hols the cart with your hands.

- START the software and at the same time release the cart: it will move along the track up to the right with uniformly accelerated motion.

- Knowing the distances s1, s2 from the left end, and knowing the times t1 and t2 when passing through the photogates 1 and 2, it is possible to determine the acceleration value:

a=2(s1-s2)t12-t22

## RESULTS AND FINDINGS

Figure 43

Distance between gates = 51cm

| T1(ms) | dT1(ms) | T2(ms) | dT2(ms) |
| --- | --- | --- | --- |
| 70.66 | 36.916 | 295.576 | 16.036 |
| 62.11 | 73.403 | 619.412 | 33.912 |

Figure 44

A1= 2(20.88)/-224.916

=-0.1857

A2 = 2(39.491)/-557.302

=-0.1417

The cart was decelerating since the rate of acceleration was negative.

# Experiment 4

## METHODOLOGY

### procedure

- Using a scale, determine the mass m1 of the cart

- Evaluate as in the previous experience the acceleration a1.

- Add an additional weight of 100gr to the cart: evaluate the new acceleration a2

- Repeat the previous operation adding another 100gr and evaluating the acceleration a3

- The mass in motion is equal to the sum of that of the cart plus that of the loaded weight hanger.

- Please enter the data in table for F = cost

Since with the same driving force the product ma remains constant, it can be said that the acceleration is inversely proportional to the mass

## RESULTS AND FINDINGS

Figure 45

Figure 46

weight of cart= 0.65N

| mass | T1 | dT1 | T2 | dT2 |
| --- | --- | --- | --- | --- |
| 0.165 | 571.585 | 162.397 | 1358.26 | 83.624 |
| 0.175 | 240.594 | 92.636 | 765.094 | 44.329 |
| 0.185 | 56.937 | 72.817 | 592.54 | 34.413 |
| 0.215 | 95.346 | 50.419 | 406.881 | 22.874 |

Figure 47

The more the mass, the faster the acceleration and speed of the cart

## REFERENCES

Coplanar Force Resolution. (n.d.). Retrieved March 2, 2022, from http://web.mit.edu/4.441/1_lectures/1_lecture7/1_lecture7.html

Determining the Coefficient of Friction—Succeed in Physical Science. (n.d.). 10.

Friction—Definition, Types of Friction, Applications & Videos. (n.d.). BYJUS. Retrieved March 2, 2022, from https://byjus.com/physics/friction/

What is a Moment? (n.d.). Retrieved March 2, 2022, from https://web.mit.edu/4.441/1_lectures/1_lecture5/1_lecture5.html